IT0019R Monte Martano

# Annex 5	Overview of sampling and analytical methods 2023

| Country: Armenia |  | Main components- EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge | AM0001R | Meteorological station | every event | By volume |
| Sulphate | AM0001R | Wet-only | every event | Ion chromatography |
| Nitrate | AM0001R | Wet-only | every event | Ion chromatography |
| Ammonium | AM0001R | Wet-only | every event | Spectrophotometric, by Nessler reagent |
| Magnesium | AM0001R | Wet-only | every event | ICP-MS |
| Sodium | AM0001R | Wet-only | every event | ICP-MS |
| Chloride | AM0001R | Wet-only | every event | Ion chromatography |
| Calcium | AM0001R | Wet-only | every event | ICP-MS |
| Potassium | AM0001R | Wet-only | every event | ICP-MS |
| Conductivity | AM0001R | Wet-only | every event | Conductivity meter |
| pH | AM0001R | Wet-only | every event | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | AM0001R | KOH-impregnated Whatman 40 filter 20–25 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | AM0001R | NaI-impregnated glass sinters, 0.6 m3/day | Daily | Spectrophotometric, Griess method |
| Nitric acid | AM0001R | KOH-impregnated Whatman 40 filter 20–25 m3/day | Daily | Ion chromatography |
| Ammonia | AM0001R | Oxalic acid-impregnated Whatman 40 filter, 20–25 m3/day | Daily | Spectrophotometric, Nessler method |
| Sulphate | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | Ion chromatography |
| Nitrate | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | Ion chromatography |
| Ammonium | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | Spectrophotometric, Nessler method |
| Sodium | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | ICP-MS |
| Calcium | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | ICP-MS |
| Magnesium | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | ICP-MS |
| Potassium | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | ICP-MS |
| Chloride | AM0001R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20–25 m3/day | Daily | Ion chromatography |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate | AM0001R | KOH-impregnated Whatman 40 filter + Teflon filter, 20–25 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | AM0001R | Oxalic acid-impregnated Whatman 40 filter +Teflon filter, 20–25 m3/day | Daily | Spectrophotometric, Nessler method |

| Country: Austria |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, off |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | All | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |
| Nitrogen dioxide | All | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6thrd day | Ion chromatography EN ISO 10304-1 |
| Nitrate | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 10304-1 |
| Ammonium | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 14911 |
| Sodium | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 14911 |
| Calcium | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 14911 |
| Magnesium | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 14911 |
| Potassium | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 14911 |
| Chloride | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Ion chromatography EN ISO 10304-1 |
| EC | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Thermal method EUSAAR protocol, optical correction (transmission) EN 16909:2017 |
| OC | AT0002R | High Volume Sampler, quartz fibre filters with binder, 720 m3/day, EN 12341 | Every 6th day | Thermal method EUSAAR protocol, optical correction (transmission) EN 16909:2017 |
| PM10 | AT0002R | High Volume Sampler, glass fibre filters with binder, 720 m3/day, every 3rd day quartz fibre filter (EN 12341) | Daily | Micro balance |
| PM10 | AT0005R, AT0048R | High Volume Sampler, glass fibre filters with binder, 720 m3/day, (EN 12341) | Every 3rd day | Micro balance |
| PM2.5 | AT0002R | High Volume Sampler, glass fibre filters with binder, 720 m3/day, , every 3rd day quartz fibre filter(EN 14907) | Daily | Micro balance |
| PM1 | AT0002R | High Volume Sampler, glass fibre filters with binder, 720 m3/day, weighing acc. EN 12341 | Every 3rd day | Micro balance |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Belarus |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  | Bulk |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  | Bulk | Daily | Turbidimetry |
| Nitrate |  | Bulk | Daily | Photometry |
| Ammonium |  | Bulk | Daily | Photometry with Nessler reactive |
| Magnesium |  | Bulk | Daily | AAS |
| Sodium |  | Bulk | Daily | AAS |
| Chloride |  | Bulk | Daily | Mercurimetric |
| Calcium |  | Bulk | Daily | AAS |
| Potassium |  | Bulk | Daily | AAS |
| Conductivity |  | Bulk | Daily | Conductivity meter |
| pH |  | Bulk | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide |  |  |  |  |
| Sulphur dioxide |  |  |  |  |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Belgium - ISSeP |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | BE0007R | Wet-only | Daily |  |
| Precipitation amount, official gauge | BE0007R | Meteorological station | Hourly |  |
| Sulphate | BE0007R | Wet-only | Daily | Ion chromatography |
| Nitrate | BE0007R | Wet-only | Daily | Ion chromatography |
| Ammonium | BE0007R | Wet-only | Daily | Ion chromatography |
| Magnesium | BE0007R | Wet-only | Daily | Ion chromatography |
| Sodium | BE0007R | Wet-only | Daily | Ion chromatography |
| Chloride | BE0007R | Wet-only | Daily | Ion chromatography |
| Calcium | BE0007R | Wet-only | Daily | Ion chromatography |
| Potassium | BE0007R | Wet-only | Daily | Ion chromatography |
| Conductivity | BE0007R | Wet-only | Daily | pH conductivity meter |
| pH | BE0007R | Wet-only | Daily | pH conductivity meter |
| Air |  |  |  |  |
| Sulphur dioxide | BE0007R | Filterpack | Daily | Ion chromatography |
| Nitrogen dioxide | BE0007R | Monitor | Hourly | Chemiluminescence |
| Nitric acid | BE0007R | Filterpack | Daily | Ion chromatography |
| Ammonia | BE0007R | Filterpack | Daily | Ion chromatography |
| Sulphate | BE0007R | Filterpack | Daily | Ion chromatography |
| Nitrate | BE0007R | Filterpack | Daily | Ion chromatography |
| Ammonium | BE0007R | Filterpack | Daily | Ion chromatography |
| Sodium | BE0007R | Filterpack | Daily | Ion chromatography |
| Calcium | BE0007R | Filterpack | Daily | Ion chromatography |
| Magnesium | BE0007R | Filterpack | Daily | Ion chromatography |
| Potassium | BE0007R | Filterpack | Daily | Ion chromatography |
| Chloride | BE0007R | Filterpack | Daily | Ion chromatography |
| PM10 | BE0007R | Monitor | Hourly | Dust monitor GRIMM 180 |
| PM2.5 | BE0007R | Monitor | Hourly | Dust monitor GRIMM 180 |
| Sum of nitric acid and nitrate | BE0007R | Filterpack | Daily | Ion chromatography |
| Sum of ammonia and ammonium | BE0007R | Filterpack | Daily | Ion chromatography |
| Ozone | BE0007R | Monitor | Hourly | Ultraviolet absorption |
| EC/OC PM2.5 | BE0007R | Low volume sampler | Daily | Thermal optical analysis |

| Country: Belgium - VMM |  | Main components - EMEP |  | Year: 2023 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Station |  | Sampling |  | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |  |  |
| Precipitation amount | BE0014R |  | Wet-only sampler |  | 2 weeks |  |
| Precipitation amount, official gauge | BE0014R |  | precipitation gauge |  |  |  |
| Sulphate | BE0014R |  | Wet-only sampler |  | 2 weeks | ion chromatography – from 18/12/23 Spectrophotmetric |
| Nitrate | BE0014R |  | Wet-only sampler |  | 2 weeks | ion chromatography – from 18/12/23 Spectrophotmetric |
| Ammonium | BE0014R |  | Wet-only sampler |  | 2 weeks | ion chromatography – from 18/12/23 Spectrophotmetric |
| Magnesium | BE0014R |  | Wet-only sampler |  | 2 weeks | ICP-AES – from 18/12/23 ICP-MS |
| Sodium | BE0014R |  | Wet-only sampler |  | 2 weeks | ICP-AES – from 18/12/23 ICP-MS |
| Chloride | BE0014R |  | Wet-only sampler |  | 2 weeks | ion chromatography – from 18/12/23 Spectrophotmetric |
| Calcium | BE0014R |  | Wet-only sampler |  | 2 weeks | ICP-AES – from 18/12/23 ICP-MS |
| Potassium | BE0014R |  | Wet-only sampler |  | 2 weeks | ICP-AES – from 18/12/23 ICP-MS |
| Conductivity | BE0014R |  | Wet-only sampler |  | 2 weeks | Conductivity probe |
| pH | BE0014R |  | Wet-only sampler |  | 2 weeks | Combined glass electrode |
| Acidity |  |  |  |  |  |  |
| Air |  |  |  |  |  |  |
| Sulphur dioxide |  |  |  |  |  |  |
| Nitrogen dioxide | BE0011R, BE0013R |  | Monitor |  | Half hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |  |  |
| Ammonia | BE0014R |  | Passive sampler |  | 4 weeks | ion chromatography – from 19/12/23 Spectrophotmetric |
| Sulphate |  |  |  |  |  |  |
| Nitrate |  |  |  |  |  |  |
| Ammonium |  |  |  |  |  |  |
| Sodium |  |  |  |  |  |  |
| Calcium |  |  |  |  |  |  |
| Magnesium |  |  |  |  |  |  |
| Potassium |  |  |  |  |  |  |
| Chloride |  |  |  |  |  |  |
| PM10 |  |  |  |  |  |  |
| PM2.5 |  |  |  |  |  |  |
| PM1 |  |  |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |  |  |

| Country: Croatia |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All |  |  |  |
| Precipitation amount, official gauge |  | Rain gauge | Daily |  |
| Sulphate | All | Bulk | Daily | Ion chromatography |
| Nitrate | All | Bulk | Daily | Ion chromatography |
| Ammonium | All | Bulk | Daily | Ion chromatography |
| Magnesium | All | Bulk | Daily | Ion chromatography |
| Sodium | All | Bulk | Daily | Ion chromatography |
| Chloride | All | Bulk | Daily | Ion chromatography |
| Calcium | All | Bulk | Daily | Ion chromatography |
| Potassium | All | Bulk | Daily | Ion chromatography |
| Conductivity | All | Bulk | Daily | Conductivity meter |
| pH | All | Bulk | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide |  |  |  |  |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 | HR0002R | Low volume sampler, Teflon filter 47 mm, 55 m3/day, EN 12341 | Daily | Gravimetric |
| PM2.5 | HR0002R | Low volume sampler, Teflon filter 47 mm, 55 m3/day, EN 12341 | Daily | Gravimetric |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Cyprus |  | Main components - EMEP | Year: 2023 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |  |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |  |
| Sulphate |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Ammonium |  |  |  |  |  |
| Magnesium |  |  |  |  |  |
| Sodium |  |  |  |  |  |
| Chloride |  |  |  |  |  |
| Calcium |  |  |  |  |  |
| Potassium |  |  |  |  |  |
| Conductivity |  |  |  |  |  |
| pH |  |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | CY02 | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |  |
| Nitrogen dioxide | CY02 | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |  |
| Nitric acid |  |  |  |  |  |
| Ammonia |  |  |  |  |  |
| Carbon Monoxide | CY02 | Non – Dispercive Infrared Sprectoscopy (NDIR) | Hourly | NDIR |  |
| Sulphate PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Nitrate PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Ammonium PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Sodium PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Calcium PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Magnesium PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Potassium PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| Chloride PM2.5 | CY02 | Low volume sampler | Daily | Ion Chromatography |  |
| PM10 | CY02 | High volume sampler | Daily | Gravimetric |  |
| PM2.5 | CY02 | Low volume sampler | Daily | Gravimetric |  |
| PM1 |  |  |  |  |  |
| OC/EC in PM2,5 | CY02 | Low volume sampler | Daily | OC EC Lab Instrument, Model 5. Sunset Laboratory Inc. EUSAAR 2 temperature program |  |

| Country: Czech Republic |  | Main components - EMEP | Year: 2023 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |  |
| Precipitation |  |  |  |  |
| Precipitation amount, official gauge | All | Meteorological Station | Daily | Automatically gauge |  |
| Fluoride | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | Ion Chromatography |  |
| Sulphate | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | Ion chromatography |  |
| Nitrate | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | Ion chromatography |  |
| Ammonium | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | Spectrophotometric, Indophenol method, SMA-Berth |  |
| Magnesium | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | F-AAS |  |
| Sodium | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | F-AAS |  |
| Chloride | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | Ion chromatography |  |
| Calcium | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | F-AAS |  |
| Potassium | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | F-AAS |  |
| Conductivity | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | Conductivity electrode |  |
| pH | All | Wet-only (daily) at CZ03, (weekly) at CZ05 | Daily, weekly | pH electrode |  |
| Air |  |  |  |  |
| Sulphur dioxide | CZ3,CZ5 | KOH-impregnated Whatman 40 filter 47 mm, 20 m3/day | Daily, CZ5 indicative(6days) | Ion chromatography |  |
| Sulphur dioxide | CZ3 | UV-fluorescence - monitor | Hourly | UV-fluorescence |  |
| Carbon monoxide | CZ3 | IR corel. absorption spectrometry | Hourly | IRABS, corel. absorption spectrometry |  |
| Nitrogen dioxide | CZ3 | Chemiluminescence - monitor | Hourly | Chemiluminescence |  |
| Nitrogen monoxide | CZ3 | Chemiluminescence - monitor | Hourly | Chemiluminescence |  |
| Sum of nitric acid and nitrate | CZ3,CZ5 | Whatman filter + KOH-impregnated Whatman 40 filter 47 mm, 20 m3/day | Daily, CZ5 indicative(6days) | Ion Chromatography |  |
| Sum of ammonia and ammonium | CZ3,CZ5 | Whatman filter + Citric acid impregnated Whatman 40 filter 47 mm, 20 m3/day | Daily, CZ5 indicative(6days) | Spectrophotometric, Indophenol method, SMA-Berth |  |
| Sulphate | CZ3,CZ5 | Whatman 40, filter 47 mm, 20 m3 /day | Daily, CZ5 indicative(6days) | Ion chromatography |  |
| Sodium | CZ3 | Filter 47 mm, 55 m3/day | Weekly | Ion chromatography |  |
| Calcium | CZ3 | Filter 47 mm, 55 m3/day | Weekly | Ion chromatography |  |
| Magnesium | CZ3 | Filter 47 mm, 55 m3/day | Weekly | Ion chromatography |  |
| Potassium | CZ3 | Filter 47 mm, 55 m3/day | Weekly | Ion chromatography |  |
| PM10 | CZ3,CZ5 | Filter 47 mm, 55 m3/day | Every 2nd day, CZ5 daily | Gravimetry |  |
| PM10 | CZ3 | Beta absorption - monitor | Hourly | Radiometry – beta absorption |  |
| PM2,5 | CZ3 | Beta absorption - monitor | Hourly | Radiometry – beta absorption |  |
| PM2.5 | CZ3 | Filter 47 mm, 55 m3/day | Every 2nd day | Gravimetry |  |
| OC, EC in PM2.5 | CZ3 | Filter 47 mm, 24 m3/day | Every 2nd day | HD-FID (Thermal-optical method) |  |

| Country: Denmark |  | Main components EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly |  |
| Precipitation amount, off gauge |  |  |  |  |
| Sulphate | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Nitrate | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Ammonium | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | ISO 11732 CFA (continuously flow analysis) and spectrophotometric detection |
| Magnesium | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Sodium | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Chloride | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Calcium | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Potassium | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | Ion chromatography |
| Conductivity |  |  |  |  |
| pH | DK05, DK08, DK12, DK22 | Wet-only | Two-weekly | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | DK03, DK08, DK12, DK31 | KOH-impregnated Whatman 41 filters, 58 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | DK05, DK08, DK12, DK31 | Monitor | Hourly | Chemiluminescence |
| Nitrogen oxide | DK05, DK08, DK12, DK31 | Monitor | Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia | DK03, DK08, DK12, DK31 | Oxalic acid impregnated Whatman 41, 58 m3/day | Daily | ISO 11732 CFA (continuously flow analysis) and spectrophotometric detection |
| Sulphate | DK03, DK05, DK08, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | Ion chromatography |
| Nitrate |  |  |  |  |
| Ammonium | DK03, DK08, DK12, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | ISO 11732 CFA (continuously flow analysis) and spectrophotometric detection |
| Sodium | DK03, DK08, DK12, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | Ion chromatography |
| Calcium | DK03, DK08, DK12, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | Ion chromatography |
| Magnesium | DK03, DK08, DK12, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | Ion chromatography |
| Potassium | DK03, DK08, DK12, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | Ion chromatography |
| Chloride | DK03, DK08, DK12, DK31 | Millipore RAWP 1.2 mm, 58 m3/day | Daily | Ion chromatography |
| PM10 | DK05, DK12 | Low volume sampling | Daily | Gravimetric |
| PM2.5 | DK12 | Low volume sampling | Daily | Gravimetric |
| Sum of nitric acid and nitrate | DK03, DK08, DK12, DK31 | Aerosol filter as for sulphate + KOH-impregnated Whatman 41, 58 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | DK05, DK08, DK12, DK22 |  |  | Replaced by separate measurements of ammonia and ammonium |

| Country: Estonia |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All | Bulk | Weekly |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | All | Bulk | Weekly | Ion chromatography |
| Nitrate | All | Bulk | Weekly | Ion chromatography |
| Ammonium | All | Bulk | Weekly | Ion chromatography |
| Magnesium | All | Bulk | Weekly | Ion chromatography |
| Sodium | All | Bulk | Weekly | Ion chromatography |
| Chloride | All | Bulk | Weekly | Ion chromatography |
| Calcium | All | Bulk | Weekly | Ion chromatography |
| Potassium | All | Bulk | Weekly | Ion chromatography |
| Conductivity | All | Bulk | Weekly | Conductivity meter |
| pH | All | Bulk | Weekly | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | All | Instrumental: UV fluorescence | Daily/Hourly | UV fluorescence |
| Nitrogen dioxide | All | Instrumental: Chemiluminescence | Daily/Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | EE09 | Filter pack | Daily |  |
| Nitrate | EE09 | Filter pack | Daily |  |
| Ammonium | EE09 | Filter pack | Daily |  |
| Sodium | EE09 | Filter pack | Daily |  |
| Calcium | EE09 | Filter pack | Daily |  |
| Magnesium | EE09 | Filter pack | Daily |  |
| Potassium | EE09 | Filter pack | Daily |  |
| Chloride | EE09 | Filter pack | Daily |  |
| PM10 | EE09 | High Volume Sampler | Weekly | Gravimetric |
| PM2.5 | All |  | Daily | β-ray absorption |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Finland |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All | NILU bulk sampler | Weekly |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | All | NILU bulk sampler | Weekly | Ion chromatography |
| Nitrate | All | NILU bulk sampler | Weekly | Ion chromatography |
| Ammonium | All | NILU bulk sampler | Weekly | Ion chromatography |
| Magnesium | All | NILU bulk sampler | Weekly | Ion chromatography |
| Sodium | All | NILU bulk sampler | Weekly | Ion chromatography |
| Chloride | All | NILU bulk sampler | Weekly | Ion chromatography |
| Calcium | All | NILU bulk sampler | Weekly | Ion chromatography |
| Potassium | All | NILU bulk sampler | Weekly | Ion chromatography |
| Conductivity | All | NILU bulk sampler | Weekly | Conductivity meter |
| pH | All | NILU bulk sampler | Weekly | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | All | NaOH-impregnated Whatman 40 filters, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Sulphur dioxide | FI18 | UV-fluorescence - monitor | Hourly | UV-fluorescence |
| Nitrogen dioxide | All | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |
| Nitric acid | All | NaOH-impregnated Whatman 40 filters, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Ammonia | All | Oxalic acid-impregnated Whatman 40 filters, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Sulphate | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Nitrate | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Ammonium | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Sodium | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Calcium | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Magnesium | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Potassium | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Chloride | All | Teflon filter, Millipore Fluoropore 3 µm, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| PM10 | All | Instrumental: beta-ray attenuation | Hourly | Beta-ray attenuation monitor |
| PM2.5 | All | Instrumental: beta-ray attenuation | Hourly | Beta-ray attenuation monitor |
| Sum of nitric acid and nitrate | All | Aerosol filter as for sulphate + NaOH impregnated Whatman 40 filter, 24 m3/day | Daily/Weekly1) | Ion chromatography |
| Sum of ammonia and ammonium | All | Aerosol filter as for sulphate + oxalic acid impregnated Whatman 40 filter, 24 m3/day | Daily/Weekly1) | Ion chromatography |

1) Daily: FI09 and FI17 and FI36; Weekly: FI22 and FI37

| Country: France |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily |  |
| Precipitation amount, official gauge | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Tipping bucket rain gauge | Daily |  |
| Sulphate | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Nitrate | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Ammonium | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Magnesium | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Sodium | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Chloride | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Calcium | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Potassium | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Ion chromatography |
| Conductivity | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | Conductivity meter |
| pH | FR08, FR09, FR10, FR13, FR14, FR15, FR16, FR17, FR18 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Nitrogen dioxide NO2/NO/NOx | FR09, FR13, FR15, FR30 | Instrumental: Chemiluminescence, trace level | Hourly | Chemiluminescence |
| Sulphate | FR08, FR09 FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Nitrate | FR08, FR09 FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Ammonium | FR08, FR09 FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Sodium | FR08, FR09, FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Calcium | FR08, FR09, FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Magnesium | FR08, FR09, FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Potassium | FR08, FR09, FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| Chloride | FR08, FR09, FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h Every 6 days | Ion chromatography |
| PM10 | FR08, FR09, FR10, FR13, FR14, FR15, FR18, FR23, FR28 | TEOM FDMS, MP101M | Hourly | TEOM FDMS, MP101M |
| PM2.5 | FR08, FR09, FR13, FR15, FR18, FR23, FR25, FR28 | TEOM FDMS, MP101M | Hourly | TEOM FDMS, MP101M |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |
| EC/OC | FR08, FR09, FR13, FR23, FR25, FR28 | TISSUQUARTZ 2500QAT-UP, PM2.5, 720m3/day | 24h every 6 days | Thermo optical, EUSAAR 2 protocol |

| Country: Georgia |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | GE01 |  | 24h every 3 days |  |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia | GE01 |  | 24h every 3 days |  |
| Sulphate | GE01 |  | 24h every 3 days | IC |
| Nitrate | GE01 |  | 24h every 3 days | IC |
| Ammonium | GE01 |  | 24h every 3 days | Spectrophotometry |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride | GE01 |  | 24h every 3 days | IC |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate | GE01 |  | 24h every 3 days |  |
| Sum of ammonia and ammonium | GE01 |  | 24h every 3 days |  |

| Country: Germany |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Gravimetric by weight |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Nitrate | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Ammonium | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Magnesium | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Sodium | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Chloride | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Calcium | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Potassium | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Ion chromatography |
| Conductivity | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | Conductivity meter |
| pH | DE01, DE02, DE03, DE07, DE08, DE09 | Daily wet only at DE02, DE03 and DE07, weekly wet-only at the other sites | Daily / weekly | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | DE01, DE02, DE03, DE07, DE08, DE09 | Monitor (trace level instrument) | hourly | UV fluorescence |
| Nitrogen dioxide | DE01, DE02, DE03, DE07, DE08, DE09 | Monitor | hourly | chemiluminescence_ photolytic |
| Nitric acid | DE02, DE03, DE07 | KOH-impregnated Whatman 40 filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Ammonia | DE02, DE03, DE07 | Oxalic acid-impregnated Whatman 40 filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Ammonia | DE01, DE02, DE03, DE07, DE08, DE09 | Low volume denuder | Weekly | Spectrophotometry/FIA |
| Sulphate | DE02, DE03, DE07 | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Nitrate | DE02, DE03, DE07 | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Ammonium |  | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Sodium |  | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Calcium |  | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Magnesium |  | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Potassium |  | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| Chloride | DE02, DE03, DE07 | Teflon filter, 22 m3/day (Filterpack) | Daily | Ion chromatography |
| PM10 | DE01, DE02, DE03, DE07, DE08, DE09 | Digitel High Volume Sampler DHA 80, glass fibre filters ø15 cm, Machery Nagel MN 85/90 | Daily | Gravimetric by weight |
| PM2.5 | DE02, DE03, DE07, DE08 | Digitel High Volume Sampler DHA 80, glass fibre filters ø15 cm, Machery Nagel MN 85/90 | Daily | Gravimetric by weight |
| PM1 | DE02 | Digitel High Volume Sampler DHA 80, glass fibre filters ø15 cm, Machery Nagel MN 85/90 | Daily | Gravimetric by weight |
| Sum of nitric acid and nitrate | DE02, DE03, DE07 | Filter pack method | Daily | Ion chromatography |
| Sum of ammonia and ammonium | DE02, DE03, DE07 | Filter pack method | Daily | Ion chromatography |
| Sulphate in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Nitrate in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Ammonium in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Sodium in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Calcium in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Magnesium in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Potassium in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| Chloride in PM2.5 | DE01, DE02, DE03, DE07, DE08, DE09 | Leckel Low Volume Sampler, 2.3 m3/hour | Every 6th day | Ion chromatography |
| EC/OC PM2.5 | DE02, DE03, DE07, DE08, DE09 | PM2.5 low volume sampler (55 m3/day), TISSUEQUARTZ 2500QAT-UP | 24 hour, once every 6 days (60 samples per year) | Thermal optical, EUSAAR-2 Protocol, |

| Country: Greece |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | GR01 | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |
| Nitrogen dioxide | GR01 | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 | GR01 | Instrumental: beta gauge | Hourly | Beta radiation attenuation |
| PM2.5 | GR01 | Instrumental: beta gauge | Hourly | Beta radiation attenuation |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Hungary |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | HU02 | Wet-only | Daily |  |
| Precipitation amount, official gauge | HU02 | Wet-only | Daily |  |
| Sulphate | HU02 | Wet-only | Daily | Ion chromatography |
| Nitrate | HU02 | Wet-only | Daily | Ion chromatography |
| Ammonium | HU02 | Wet-only | Daily | Spectrophotometric, Indophenol method |
| Magnesium | HU02 | Wet-only | Daily | Atomic absorption method (flame) |
| Sodium | HU02 | Wet-only | Daily | Atomic absorption method (flame) |
| Chloride | HU02 | Wet-only | Daily | Ion chromatography |
| Calcium | HU02 | Wet-only | Daily | Atomic absorption method (flame) |
| Potassium | HU02 | Wet-only | Daily | Atomic absorption method (flame) |
| Conductivity | HU02 | Wet-only | Daily | Conductivity meter |
| pH | HU02 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | HU02 | KOH-impregnated Whatman 40 filter, ~21 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | HU02 | Monitor | Hourly | Chemiluminescence |
| Nitric acid | HU02 | KOH-impregnated Whatman 40 filter, ~21 m3/day | Daily | Ion chromatography |
| Ammonia | HU02 | Citric-acid impregnated Whatman 40 filter, ~21 m3/day | Daily | Spectrophotometric, Indophenol method |
| Sulphate | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Ion chromatography |
| Nitrate | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Ion chromatography |
| Ammonium | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Spectrophotometric, Indophenol method |
| Sodium | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Atomic absorption method (flame) |
| Calcium | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Atomic absorption method (flame) |
| Magnesium | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Atomic absorption method (flame) |
| Potassium | HU02 | Teflon filter, Millipore Fluoropore, 1 mm, ~21 m3/day | Daily | Atomic absorption method (flame) |
| Chloride |  |  |  |  |
| PM10 mass | HU02 | PM10-monitor | Hourly | Beta-ray-absorption |
| PM2.5 mass | HU02 | DHA-80 high volume sampler | Daily | Gravimetry |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Iceland |  | Main components- EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | IS02 | NILU bulk sampler | Daily | By volume |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | IS02 | NILU bulk sampler | Daily | ICP-OES |
| Nitrate | IS02 | NILU bulk sampler | Daily | Spectrophotometry by FIA |
| Ammonium |  |  |  |  |
| Magnesium | IS02 | NILU bulk sampler | Daily | ICP-OES |
| Sodium | IS02 | NILU bulk sampler | Daily | ICP-OES |
| Chloride | IS02 | NILU bulk sampler | Daily | ICP-OES |
| Calcium | IS02 | NILU bulk sampler | Daily | ICP-OES |
| Potassium | IS02 | NILU bulk sampler | Daily | ICP-OES |
| Conductivity | IS02 | NILU bulk sampler | Daily | Conductivity meter |
| pH | IS02 | NILU bulk sampler | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | IS02 | KOH impregnated Whatman 40 filter, 30 m3/day | Daily | ICP-OES |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | IS02 | Whatman 40 filter, 30 m3/day, prefilter for aerosol | Daily | ICP-OES |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium | IS02 | Whatman 40 filter, 30 m3/day, prefilter for aerosol | Daily | ICP-OES |
| Calcium | IS02 | Whatman 40 filter, 30 m3/day, prefilter for aerosol | Daily | ICP-OES |
| Magnesium | IS02 | Whatman 40 filter, 30 m3/day, prefilter for aerosol | Daily | ICP-OES |
| Potassium | IS02 | Whatman 40 filter, 30 m3/day, prefilter for aerosol | Daily | ICP-OES |
| Chloride | IS02 | Whatman 40 filter, 30 m3/day, prefilter for aerosol | Daily | ICP-OES |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Ireland - Met Éireann (IE01) |  | Main components - EMEP | Year: 2023 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |  |
| Precipitation |  |  |  |  |
| Precipitation amount | IE01,IE05,IE09 | Wet-only | Daily |  |  |
| Precipitation amount, official gauge | IE01,IE05,IE09 | Rain gauge | Daily |  |  |
| Sulphate | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Nitrate | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Ammonium | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Magnesium | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Sodium | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Chloride | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Calcium | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Potassium | IE01,IE05,IE09 | Wet-only | Daily | Ion chromatography |  |
| Conductivity | IE01,IE05,IE09 | Wet-only | Daily | Conductivity meter |  |
| pH | IE01,IE05,IE09 | Wet-only | Daily | pH meter |  |
| Air |  |  |  |  |
| Sulphur dioxide | IE01 | KOH-impregnated Whatman 40 filter, 25 m3/day | Daily | Ion chromatography |  |
| Nitrogen dioxide | IE01 | NaI method (glass sinter) 0.7 m3/day | Daily | Spectrophotometric, EMEP Manual 4.11 |  |
| Nitric acid | IE01 | KOH-impregnated Whatman 40 filter, 25 m3/day | Daily | Ion chromatography |  |
| Ammonia | IE01 | Oxalic acid impregnated filter | Daily | Ion chromatography |  |
| Sulphate | IE01,IE05,IE06,IE08 | KOH-impregnated Whatman 40 filter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Nitrate | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Ammonium | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Sodium | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Calcium | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Magnesium | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Potassium | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| Chloride | IE01,IE05,IE06,IE08 | Whatman 40 filter, 47 mm diameter, 25 m3/day (IE01) | Daily | Ion chromatography |  |
| PM10 |  |  |  |  |  |
| PM2.5 |  |  |  |  |  |
| Sum of nitric acid and nitrate | IE01 | Aerosol filter as for sulphate + KOH impregnated filter as for SO2, 20-25 m3/day | Daily | Ion chromatography |  |
| Sum of ammonia and ammonium | IE01 | Aerosol filter as for sulphate + citric acid impregnated filter, 20-25 m3/day | Daily | Ion chromatography |  |

| Country: Italy - JRC (IT04) |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation amount | IT04 | Wet-only | Daily | Sampler gauge |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | IT04 | Wet-only | Daily | Ion chromatography |
| Nitrate | IT04 | Wet-only | Daily | Ion chromatography |
| Ammonium | IT04 | Wet-only | Daily | Ion chromatography |
| Magnesium | IT04 | Wet-only | Daily | Ion chromatography |
| Sodium | IT04 | Wet-only | Daily | Ion chromatography |
| Chloride | IT04 | Wet-only | Daily | Ion chromatography |
| Calcium | IT04 | Wet-only | Daily | Ion chromatography |
| Potassium | IT04 | Wet-only | Daily | Ion chromatography |
| Conductivity | IT04 | Wet-only | Daily | Conductivity meter |
| pH | IT04 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | IT04 | Instrumental: UV-fluorescence | Daily | UV-fluorescence |
| Nitrogen dioxide | IT04 | Instrumental: Chemiluminescence | Daily | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Nitrate | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Ammonium | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Sodium | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Calcium | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Magnesium | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Potassium | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| Chloride | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Ion chromatography |
| PM10 |  |  |  |  |
| PM2.5 | IT04 | PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Weighing at 20% RH |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |
| EC/OC | IT04 | AirMonitors Denuder, PALL Life Sciences QFF (type TISSUEQUARTZ 2500QAT-UP), 24 m3/day | Daily | Thermo optical, EUSAAR 2 protocol |
| Precipitation amount | IT04 | Wet-only | Daily | Sampler gauge |

| Country: Italy – CNR-ISAC (IT09) |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Acidity |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | IT0009R | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |
| Nitrogen dioxide | IT0009R | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Italy – CNR (IT01) |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | IT0001R | Wet only | Daily | By volume |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | IT0001R | Wet only | Daily | Ion chromatography |
| Nitrate | IT0001R | Wet only | Daily | Ion chromatography |
| Ammonium | IT0001R | Wet only | Daily | Ion chromatography |
| Magnesium | IT0001R | Wet only | Daily | Ion chromatography |
| Sodium | IT0001R | Wet only | Daily | Ion chromatography |
| Chloride | IT0001R | Wet only | Daily | Ion chromatography |
| Calcium | IT0001R | Wet only | Daily | Ion chromatography |
| Potassium | IT0001R | Wet only | Daily | Ion chromatography |
| Conductivity | IT0001R | Wet only | Daily | Conductivity meter |
| pH | IT0001R | Wet only | Daily | pH meter |
| Acidity |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | IT0001R | Filterpack | Daily | Ion chromatography |
| Nitrogen dioxide | IT0001R | Monitor | Hourly | chemiluminescence_molybdenum |
| Nitric acid | IT0001R | Filterpack | Daily | Ion chromatography |
| Ammonia | IT0001R | Filterpack | Daily | Ion chromatography |
| Sulphate | IT0001R | Filterpack | Daily | Ion chromatography |
| Nitrate | IT0001R | Filterpack | Daily | Ion chromatography |
| Ammonium | IT0001R | Filterpack | Daily | Ion chromatography |
| Sodium | IT0001R | Filterpack | Daily | Ion chromatography |
| Calcium | IT0001R | Filterpack | Daily | Ion chromatography |
| Magnesium | IT0001R | Filterpack | Daily | Ion chromatography |
| Potassium | IT0001R | Filterpack | Daily | Ion chromatography |
| Chloride | IT0001R | Filterpack | Daily | Ion chromatography |
| PM10 | IT0001R | Low vol, FAI Instruments SWAM 5A Dual Channel | Daily | Beta attenuation |
| PM2.5 | IT0001R | Low vol, FAI Instruments SWAM 5A Dual Channel | Daily | Beta attenuation |
| EC/OC PM2.5 | IT0001R | Low vol, FAI Instruments SWAM 5A Dual Channel | Daily | Thermo optical |
| Sum of nitric acid and nitrate | IT0001R | Filterpack | Daily | Ion chromatography |
| Sum of ammonia and ammonium | IT0001R | Filterpack | Daily | Ion chromatography |

| Country: Italy - Arpa Umbria (IT19) |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Acidity |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide |  |  |  |  |
| Nitrogen dioxide | IT0019R | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Nitrate | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Ammonium | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Sodium | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Calcium | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Magnesium | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Potassium | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| Chloride | IT0019R | Low volume sampler Quartz Filter | Daily | Ion chromatography |
| PM10 | IT0019R | Low volume sampler Quartz Filter | Daily | Beta radiation attenuation |
| PM2.5 | IT0019R | Low volume sampler Quartz Filter | Daily | Beta radiation attenuation |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |
| EC/OC PM10 | IT0019R | Low volume sampler Quartz Filter | Daily | Thermal-optical, EUSAAR 2 protocol |

| Country: Kazakhstan |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide |  |  |  |  |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate PM10 | KZ01 |  | Daily | IC |
| Nitrate PM10 | KZ01 |  | Daily | IC |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride PM10 | KZ01 |  | Daily | IC |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Latvia |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | LV10 | Wet-only | Daily | Gravimetric |
| Precipitation amount, official gauge | LV10 | Meteorological station | Daily | Automatic Rain gauge, OTT Pluvio2 |
| Sulphate | LV10 | Wet-only | Daily | Ion chromatography |
| Nitrate | LV10 | Wet-only | Daily | Ion chromatography |
| Ammonium | LV10 | Wet-only | Daily | Spectrophotometric, Indophenol method |
| Magnesium | LV10 | Wet-only | Daily | ICP-AES |
| Sodium | LV10 | Wet-only | Daily | ICP-AES |
| Chloride | LV10 | Wet-only | Daily | Ion chromatography |
| Calcium | LV10 | Wet-only | Daily | ICP-AES |
| Potassium | LV10 | Wet-only | Daily | ICP-AES |
| Conductivity | LV10 | Wet-only | Daily | Conductivity meter |
| pH | LV10 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | LV10 | KOH-impregnated Whatman 47 filter, 16-23 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | LV10 | NaI-impregnated glass sinters, 03-0.7 m3/day | Daily | Spectrophotometric, Griess method |
| Nitric acid | LV10 | KOH-impregnated Whatman 47 filter, 16-23 m3/day | Daily | Ion chromatography |
| Ammonia | LV10 | Oxalic acid impregnated filter, 16-23 m3/day | Daily | Spectrophotometric, Indophenol method |
| Sulphate | LV10 | Whatman 47 filter, 16-23 m3/day | Daily | Ion chromatography |
| Nitrate | LV10 | Whatman 47 filter, 16-23 m3/day | Daily | Ion chromatography |
| Ammonium | LV10 | Whatman 47 filter, 16-23 m3/day | Daily | Spectrophotometric, Indophenol method |
| Sulphate PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Nitrate PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Ammonium PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Sodium PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Calcium PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Magnesium PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Potassium PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| Chloride PM2.5 | LV10 | Teflon filter, 386.4 m3/weekly | Weekly | Ion chromatography |
| PM10 | LV10 | Low volume sampler, 2.3 m3/h, Teflon filter, 47 mm | Daily | Beta absorption |
| PM2.5 | LV10 | Low volume sampler, 2.3 m3/h, Teflon filter, 47 mm | Daily | Beta absorption |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate | LV10 | KOH-impregnated Whatman 47 filter + Whatman 47 filter, 16-23 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | LV10 | Oxalic acid impregnated filter + Whatman 47 filter, 16-23 m3/day | Daily | Spectrophotometric, Indophenol method |

| Country: Lithuania |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | LT15 | Wet-only | Daily | By weight |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | LT15 | Wet-only | Daily | Ion chromatography |
| Nitrate | LT15 | Wet-only | Daily | Ion chromatography |
| Ammonium | LT15 | Wet-only | Daily | Spectrophotometric, Indophenol method |
| Magnesium | LT15 | Wet-only | Daily | Atomic absorption method |
| Sodium | LT15 | Wet-only | Daily | Atomic emission method |
| Chloride | LT15 | Wet-only | Daily | Ion chromatography |
| Calcium | LT15 | Wet-only | Daily | Atomic absorption method |
| Potassium | LT15 | Wet-only | Daily | Atomic emission method |
| Conductivity | LT15 | Wet-only | Daily | Conductivity meter |
| pH | LT15 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | LT15 | KOH-impregnated Whatman 40 filter, 20 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | LT15 | NaI-impregnated glass sinters, 0.7 m3/day | Daily | Spectrophotometric, Griess method |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Ion chromatography |
| Nitrate | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Ion chromatography |
| Ammonium | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Spectrophotometric, Indophenol method |
| Sodium | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Atomic emission method |
| Calcium | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Atomic absorption method |
| Magnesium |  |  |  |  |
| Potassium | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Atomic emission method |
| Chloride | LT15 | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 20m3/day (Filterpack) | Daily | Ion chromatography |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate | LT15 | Aerosol filter as for sulphate + KOH impregnated Whatman 40 filter as for SO2, 20 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | LT15 | Aerosol filter as for sulphate + oxalic acid impregnated Whatman 40 filter, 20 m3/day | Daily | Spectrophotometric, Indophenol method |

| Country: Macedonia |  | Main components - EMEP | Year: 2023 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |  |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |  |
| Sulphate |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Ammonium |  |  |  |  |  |
| Magnesium |  |  |  |  |  |
| Sodium |  |  |  |  |  |
| Chloride |  |  |  |  |  |
| Calcium |  |  |  |  |  |
| Potassium |  |  |  |  |  |
| Conductivity |  |  |  |  |  |
| pH |  |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | MK07 | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |  |
| Nitrogen dioxide | MK07 | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |  |
| Nitric acid |  |  |  |  |  |
| Ammonia |  |  |  |  |  |
| Sulphate |  |  |  |  |  |
| Nitrate |  |  |  |  |  |
| Ammonium |  |  |  |  |  |
| Sodium |  |  |  |  |  |
| Calcium |  |  |  |  |  |
| Magnesium |  |  |  |  |  |
| Potassium |  |  |  |  |  |
| Chloride |  |  |  |  |  |
| PM10 | MK07 | Instrumental: beta absorption | Hourly | Beta absorption |  |
| PM2.5 |  |  |  |  |  |
| PM1 |  |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |  |

| Country: Malta |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide | MT0001R | Instrumental: UV-fluorescence monitor | Hourly | UV-fluorescence |
| Nitrogen dioxide | MT0001R | Instrumental: Chemiluminescence monitor | Hourly | Chemiluminescence (molybdenum converter) |
| Nitrogen monoxide | MT0001R | Instrumental: Chemiluminescence monitor | Hourly | Chemiluminescence (molybdenum converter) |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Moldova |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | MD13 | NILU bulk sampler | Daily | By volume |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Nitrate | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Ammonium | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Magnesium | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Sodium | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Chloride | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Calcium | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Potassium | MD13 | NILU bulk sampler | Daily | Ion chromatography |
| Conductivity | MD13 | NILU bulk sampler | Daily | Conductivity meter |
| pH | MD13 | NILU bulk sampler | Daily | pH meter; potentiometric, glass electrode |
| Air |  |  |  |  |
| Sulphur dioxide | MD13 | KOH-impregnated Whatman 40 filter 25 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Nitrate | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Ammonium | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Sodium | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Calcium | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Magnesium | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Potassium | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Chloride | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| PM10 | MD13 | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate | MD13 | Aerosol filter as for sulphate + KOH impregnated filter as for SO2, 25 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | MD13 | Aerosol filter as for sulphate + oxalic acid impregnated filter, 25 m3/day | Daily | Spectrophotometric, Indophenol method and IC |
| EC/OC |  |  |  |  |

| Country: Montenegro |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | ME08 | Wet-only | daily |  |
| Precipitation amount, official gauge | ME08 | Meteorological station | daily |  |
| Sulphate | ME08 | Wet-only | daily | Spectrophotometry |
| Nitrate | ME08 | Wet-only | daily | Spectrophotometry |
| Ammonium | ME08 | Wet-only | daily | Spectrophotometry |
| Magnesium | ME08 | Wet-only | daily |  |
| Sodium | ME08 | Wet-only | daily | Flame photometry |
| Chloride | ME08 | Wet-only | daily | Titrimetric method |
| Calcium | ME08 | Wet-only | daily | Titrimetric method |
| Potassium | ME08 | Wet-only | daily | Flame photometry |
| Conductivity | ME08 | Wet-only | daily | Conductivity meter |
| pH | ME08 | Wet-only | daily | pH meter, glass electrode |
| Air |  |  |  |  |
| Sulphur dioxide | ME08 | Absorbing solution | Daily | Spectrophotometry |
| Nitrogen dioxide | ME08 | Absorbing solution | Daily | Spectrophotometry |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| PM1 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: The Netherlands |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |  |
| Precipitation |  |  |  |  |  |
| Precipitation amount | NL091 | Wet-only | Daily |  |  |
| Precipitation amount, official gauge |  |  |  |  |  |
| Sulphate | NL091 | Wet-only | Daily | Ion chromatography |  |
| Nitrate | NL091 | Wet-only | Daily | Ion chromatography |  |
| Ammonium | NL091 | Wet-only | Daily | CFA |  |
| Magnesium | NL091 | Wet-only | Daily | HR-ICP/MS |  |
| Sodium | NL091 | Wet-only | Daily | HR-ICP/MS |  |
| Chloride | NL091 | Wet-only | Daily | Ion chromatography |  |
| Calcium | NL091 | Wet-only | Daily | HR-ICP/MS |  |
| Potassium | NL091 | Wet-only | Daily | HR-ICP/MS |  |
| Conductivity | NL091 | Wet-only | Daily | Conductivity meter |  |
| pH | NL091 | Wet-only | Daily | pH meter |  |
| Air |  |  |  |  |  |
| Sulphur dioxide | NL07,NL09,,NL91,NL644R | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |  |
| Nitrogen dioxide | NL07,NL09,NL10,NL91,NL644R | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |  |
| Nitric acid |  |  |  |  |  |
| Ammonia | NL91 | miniDOAS: open path UV differential absorption, fingerprint 205-230 nm | Hourly | DOAS |  |
| Sulphate | NL10,NL91 | Whatman QMA filter 47 mm, 55.2 m3/day | Every other day | Ion chromatography |  |
| Nitrate | NL10,NL91 | Whatman QMA filter 47 mm, 55.2 m3/day | Every other day | Ion chromatography |  |
| Ammonium | NL10,NL91 | Whatman QMA filter 47 mm, 55.2 m3/day | Every other day | CFA2 |  |
| Chloride | NL10,NL91 | Whatman QMA filter 47 mm, 55.2 m3/day | Every other day | Ion chromatography |  |
| Sodium | NL08, NL644R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 55.2 m3/day | NL08L every other day; NL644R every 4 day | HR-ICP/MS |  |
| Calcium | NL08, NL644R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 55.2 m3/day | NL08L every other day; NL644R every 4 day | HR-ICP/MS |  |
| Magnesium | NL08, NL644R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 55.2 m3/day | NL08L every other day; NL644R every 4 day | HR-ICP/MS |  |
| Potassium | NL644R | Teflon filter, Pall Zefluor 2 µm, 47 mm diameter, 55.2 m3/day | NL644R every 4 day | HR-ICP/MS |  |
| PM10 | NL07,NL09,NL10,NL91,NL644R | Instrumental: beta absorption | Hourly | Beta absorption |  |
| PM2.5 | NL09,NL10,,NL91.NL644R | Instrumental: beta absorption | Hourly | Beta absorption |  |
| Sum of nitric acid and nitrate |  |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |  |

| Country: Norway |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All | NILU bulk sampler | Daily | By volume |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | All | NILU bulk sampler | Daily | Ion chromatography |
| Nitrate | All | NILU bulk sampler | Daily | Ion chromatography |
| Ammonium | All | NILU bulk sampler | Daily | Ion chromatography |
| Magnesium | All | NILU bulk sampler | Daily | Ion chromatography |
| Sodium | All | NILU bulk sampler | Daily | Ion chromatography |
| Chloride | All | NILU bulk sampler | Daily | Ion chromatography |
| Calcium | All | NILU bulk sampler | Daily | Ion chromatography |
| Potassium | All | NILU bulk sampler | Daily | Ion chromatography |
| Conductivity | All | NILU bulk sampler | Daily | Conductivity meter |
| pH | All | NILU bulk sampler | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | All | KOH-impregnated Whatman 40 filter 25 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | All | NaI-impregnated glass sinters, 0.7 m3/day | Daily | Spectrophotometric |
| Nitric acid | All | KOH-impregnated Whatman 40 filter 25 m3/day | Daily |  |
| Ammonia | All | Oxalic acid impregnated filter, 25 m3/day | Daily |  |
| Sulphate | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Nitrate | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Ammonium | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Sodium | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Calcium | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Magnesium | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Potassium | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| Chloride | All | Teflon filter, Gelman Zefluor 2 µm, 25 m3/day | Daily | Ion chromatography |
| PM10 | NO01,NO39,NO56 | Kleinfiltergerät Whatman QM-A 47 mm | Weekly | by weight, RH 50% |
| PM2.5 | NO01,NO39,NO56 | Kleinfiltergerät Whatman QM-A 47 mm | Weekly | by weight, RH 50% |
| PM1 | NO01,NO39,NO56 | Kleinfiltergerät Whatman QM-A 47 mm | Weekly | by weight, RH 50% |
| Sum of nitric acid and nitrate | All | Aerosol filter as for sulphate + KOH impregnated filter as for SO2, 25 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | All | Aerosol filter as for sulphate + oxalic acid impregnated filter, 25 m3/day | Daily | Ion chromatography |
| EC/OC | NO01,NO39,NO56 | Kleinfiltergerät Whatman QM-A 47 mm, 55 m3/day | Weekly | Thermal optical transmission |

| Country: Poland - IMWM-NRI (PL02,PL03,PL04) |  | Main components- EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All | Bulk | Daily | By_weight |
| Precipitation amount, official gauge | All | Total | Daily | PL02,PL03 Hellman, standard gauge PL04 SEBA_Hydrometrie, automatic gauge |
| Sulphate | All | Bulk | Daily | Ion chromatography |
| Nitrate | All | Bulk | Daily | Ion chromatography |
| Ammonium | All | Bulk | Daily | Spectrophotometric, Chloramin T |
| Magnesium | All | Bulk | Daily | Atomic absorption method |
| Sodium | All | Bulk | Daily | Atomic absorption method |
| Chloride | All | Bulk | Daily | Ion chromatography |
| Calcium | All | Bulk | Daily | Atomic absorption method |
| Potassium | All | Bulk | Daily | Atomic absorption method |
| Conductivity | All | Bulk | Daily | Conductivity meter |
| pH | All | Bulk | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | All | KOH-impregnated Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric,Thorin |
| Nitrogen dioxide | All | Absorbing solution TGS, 0.7 m3/day | Daily | Spectrophotometric, Griess method |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | All | Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric,Thorin |
| Nitrate | All | Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric, Griess after hydrazine reduction |
| Ammonium | All | Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric, Chloramin T |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride | All | Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric, Thiocyanate |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| Sum of nitric acid and nitrate | All | NaF impregnated Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric, Griess after hydrazine reduction |
| Sum of ammonia and ammonium | All | Oxalic acid impregnated Whatman 40 filter, 3.5-4.2 m3/day | Daily | Spectrophotometric, Chloramin T |

| Country: Poland – IEP-NRI (PL05) |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | PL05 | Wet-only | Daily | By weight |
| Precipitation amount, official gauge | PL05 | Total | Daily | Standard rain gauge |
| Sulphate | PL05 | Wet-only | Daily | Ion chromatography |
| Nitrate | PL05 | Wet-only | Daily | Ion chromatography |
| Ammonium | PL05 | Wet-only | Daily | Spectrophotometric, Indophenol method |
| Magnesium | PL05 | Wet-only | Daily | ICP-AES |
| Sodium | PL05 | Wet-only | Daily | ICP-AES |
| Chloride | PL05 | Wet-only | Daily | Ion chromatography |
| Calcium | PL05 | Wet-only | Daily | ICP-AES |
| Potassium | PL05 | Wet-only | Daily | ICP-AES |
| Conductivity | PL05 | Wet-only | Daily | Conductivity meter |
| pH | PL05 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | PL05 | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |
| Nitrogen dioxide | PL05 | Instrumental: Chemiluminescence | Hourly | Chemiluminescence |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | PL05 | Teflon filter Millipore Fluoropore 3 µm, 16 m3/day | Daily | Capillary Electrophoresis |
| Sulphate | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| Nitrate | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| Ammonium | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Spectrophotometric, Indophenol method |
| Sodium | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Calcium | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Magnesium | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Potassium | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Chloride | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| EC/OC | PL05 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily | Thermo optical |
| PM10 | PL05 | High Volume Sampler (750 m3/day) | Daily | By weight |
| PM2.5 | PL05 | High Volume Sampler (750 m3/day) | Daily | By weight |
| Sum of nitric acid and nitrate | PL05 | Aerosol Teflon filter Millipore Fluoropore 3 µm+ KOH impregnated Whatman 40 filter, 16 m3/day | Daily | Capillary Electrophoresis |
| Sum of ammonia and ammonium | PL05 | Aerosol Teflon filter Millipore Fluoropore 3 µm + Oxalic acid impregnated Whatman 40 filter, 16 m3/day | Daily | Spectrophotometric, Indophenol method |

| Country: Poland – CLR (PL09) |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Magnesium |  |  |  |  |
| Sodium |  |  |  |  |
| Chloride |  |  |  |  |
| Calcium |  |  |  |  |
| Potassium |  |  |  |  |
| Conductivity |  |  |  |  |
| pH |  |  |  |  |
| Air |  |  |  |  |
| Sulphur dioxide |  |  |  |  |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Sulphate | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| Nitrate | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| Ammonium | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| Sodium | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Calcium | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Magnesium | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Potassium | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | ICP-AES |
| Chloride | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Daily/Weekly (anal.) | Ion chromatography |
| EC/OC | PL09 | QMA Whatman filter, 750 m3/day (PM2,5) | Every 2nd day | Thermo optical |
| PM10 | PL09 | High Volume Sampler (750 m3/day) | Daily | By weight |
| PM2.5 | PL09 | High Volume Sampler (750 m3/day) | Daily | By weight |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Russian Federation |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All | Bulk | Daily |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | All | Bulk | Daily | Ion chromatography |
| Nitrate | All | Bulk | Daily | Ion chromatography |
| Ammonium | All | Bulk | Daily | Ion chromatography |
| Magnesium | All | Bulk | Daily | Ion chromatography |
| Sodium | All | Bulk | Daily | Ion chromatography |
| Chloride | All | Bulk | Daily | Ion chromatography |
| Calcium | All | Bulk | Daily | Ion chromatography |
| Potassium | All | Bulk | Daily | Ion chromatography |
| Conductivity | All | Bulk | Daily | Conductivity meter |
| pH | All | Bulk | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | RU18 | NaOH-impregnated Whatman 40 filter, 20-25 m3/day | Daily | Ion chromatography |
| Sulphur dioxide | RU20 | NaOH-impregnated Whatman 40 filter, 20-25 m3/day | Weekly | Ion chromatography |
| Nitrogen dioxide |  |  |  |  |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | RU18 | Whatman 40 filter, 20-25 m3/day | Daily | Ion chromatography |
| Sulphate | RU20 | Whatman 40 filter, 20-25 m3/day | Weekly | Ion chromatography |
| Nitrate | RU18 | Whatman 40 filter, 20-25 m3/day | Daily | Ion chromatography |
| Nitrate | RU20 | Whatman 40 filter, 20-25 m3/day | Weekly | Ion chromatography |
| Ammonium | RU18 | Whatman 40 filter, 20-25 m3/day | Daily | Ion chromatography |
| Ammonium | RU20 | Whatman 40 filter, 20-25 m3/day | Weekly | Ion chromatography |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 |  |  |  |  |
| PM2.5 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Serbia |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount |  |  |  |  |
| Precipitation amount, official gauge | RS05 | Meteorological rain gauge | Daily |  |
| Sulphate | RS05 | Bulk | Daily | Ion chromatography |
| Nitrate | RS05 | Bulk | Daily | Ion chromatography |
| Ammonium | RS05 | Bulk | Daily | Ion chromatography |
| Magnesium | RS05 | Bulk | Daily | Ion chromatography |
| Sodium | RS05 | Bulk | Daily | Ion chromatography |
| Chloride | RS05 | Bulk | Daily | Ion chromatography |
| Calcium | RS05 | Bulk | Daily | Ion chromatography |
| Potassium | RS05 | Bulk | Daily | Ion chromatography |
| Conductivity | RS05 | Bulk | Daily | Conductivity meter |
| pH | RS05 | Bulk | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | RS05 | Absorbing solution H2O2, 1.5-2.5 m3/day | Daily | Thorin Spectrophotometric method |
| Nitrogen dioxide | RS05 | Absorbing solution NaOH, 1.5-2.5 m3/day | Daily | Modified Griess Saltzman method |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate |  |  |  |  |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium |  |  |  |  |
| Calcium |  |  |  |  |
| Magnesium |  |  |  |  |
| Potassium |  |  |  |  |
| Chloride |  |  |  |  |
| PM10 | RS05 | Low Volume Sampler, 2.3 m3/day | Daily | Gravimetric method |
| PM2.5 |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |

| Country: Slovakia |  | Main components EMEP |  | Year: 2023 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency |  | Analysis method |  |
| Precipitation |  |  |  |  |  |
| Precipitation amount | All | Bulk: SK02: Wet-only: SK04, SK06, SK07 | Daily SK02, SK06 Weekly SK04, SK 07 |  | By weight |  |
| Precipitation amount, official gauge |  |  |  |  |  |  |
| Sulphate | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Nitrate | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Ammonium | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Magnesium | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Sodium | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Chloride | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Calcium | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Potassium | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Ion chromatography – Dionex |  |
| Conductivity | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | Conductivity meter |  |
| pH | All | Bulk: SK02 Wet-only: SK04, SK06, SK07 | Daily: SK02, SK06 Weekly: SK04, SK07 |  | pH meter |  |
| Air |  |  |  |  |  |
| Sulphur dioxide | SK02,SK06 | KOH-impregnated Whatman 40 filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Nitrogen dioxide | SK02,SK06 | Absorbing solution NaOH and guajacol, 0.5-0.6 m3/day | Daily |  | Spectrophotometric, Modified Salzman method |  |
| Nitric acid | SK02,SK06 | KOH-impregnated Whatman 40 filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Ammonia | SK06 | Citric acid-impregnated Whatman 40 filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Sulphate | SK02,SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Nitrate | SK02,SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex l |  |
| Ammonium | SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Sodium | SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Calcium | SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Magnesium | SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Potassium | SK06 | Teflon filter, filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| Chloride | SK02,SK06 | Teflon filter, 26-30 m3/day | Daily |  | Ion chromatography – Dionex |  |
| PM10 | SK04, SK06, SK07 | Low volume sampler (MicroPNS), Sartorius nitrocellulose filter, Teom | Weekly (SK06), 72 hour (SK04, SK07) |  | Gravimetric method (SK06), Teom (SK04, SK07) |  |
| TSP | SK02 | Low volume sampler | Weekly |  | Gravimetric method |  |
| EC/OC PM2.5 | SK04 | PM2.5 low volume sampler (55 m3/day) | Weekly |  | Thermal optical analysis |  |

| Country: Slovenia |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | SI08 | Wet-only | Daily | By weight |
| Precipitation amount, official gauge | SI08 | Bulk | Daily |  |
| Sulphate | SI08 | Wet-only | Daily | Ion chromatography |
| Nitrate | SI08 | Wet-only | Daily | Ion chromatography |
| Ammonium | SI08 | Wet-only | Daily | Ion chromatography |
| Magnesium | SI08 | Wet-only | Daily | Ion chromatography |
| Sodium | SI08 | Wet-only | Daily | Ion chromatography |
| Chloride | SI08 | Wet-only | Daily | Ion chromatography |
| Calcium | SI08 | Wet-only | Daily | Ion chromatography |
| Potassium | SI08 | Wet-only | Daily | Ion chromatography |
| Conductivity | SI08 | Wet-only | Daily | Conductivity meter |
| pH | SI08 | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Nitrogen dioxide | SI08 | Instrumental: Cavity-Attenuated Phase-Shift spectroscopy | Hourly | Cavity-Attenuated Phase-Shift spectroscopy |
| Sulphur dioxide | SI08 | Instrumental: UV-Fluorescence | Hourly | UV-Fluorescence |
| PM10 | SI08 | Low volume sampler, 2.3 m3/h, Quartz filter, 47 mm | Daily | Gravimetric method |
| PM2.5 | SI08 | Low volume sampler, 2.3 m3/h, Quartz filter, 47 mm | Daily | Gravimetric method |
| Ammonium PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Nitrate PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Sulphate PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Calcium PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Cloride PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Magnesium PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Sodium PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| Potassium PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Ion chromatography |
| EC/OC PM2.5 | SI08 | Leckel - Low volume sampler | Daily | Thermal optical analysis |

| Country: Spain - MITECO |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | All (except ES10) | Wet-only | Daily |  |
| Sulphate | All (except ES10) | Wet-only | Daily | Ion chromatography |
| Nitrate | All (except ES10) | Wet-only | Daily | Ion chromatography |
| Ammonium | All (except ES10) | Wet-only | Daily | Ion chromatography |
| Magnesium | All (except ES10) | Wet-only | Daily | ICP-AES |
| Sodium | All (except ES10) | Wet-only | Daily | ICP-AES |
| Chloride | All (except ES10) | Wet-only | Daily | Ion chromatography |
| Calcium | All (except ES10) | Wet-only | Daily | ICP-AES |
| Potassium | All (except ES10) | Wet-only | Daily | ICP-AES |
| Conductivity | All (except ES10) | Wet-only | Daily | Conductivity meter |
| pH | All (except ES10) | Wet-only | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | All | Instrumental: UV-fluorescence | Hourly | Pulsed UV-Fluorescence |
| Nitrogen dioxide/NO/NOx | All | Instrumental: Chemiluminescence | Hourly | Chemiluminiscence |
| PM10 | ES09, ES12, ES13, ES16 | Instrumental: beta-ray attenuation | Hourly | Beta-ray attenuation monitor |
| Ammonia | ES01, ES07, ES08, ES09, ES14 | Passive sampler | Weekly ES07 (Biweekly) | Visible spectrophotometry, Indophenol method |
| PM10 | All | High volume sampler | Daily | Gravimetric method |
| PM2.5 | ES01, ES06, ES07, ES08, ES09, ES10, ES11, ES12, ES13, ES14, ES16 | High volume sampler (ES01, ES07, ES08, ES09, ES10, ES12, ES14) Low volumen sampler (ES06, ES11, ES13, ES16) | Daily | Gravimetric method |
| Sulphate PM10 | All | Whatman GF/A filter, 720 m3/day (ES05,ES06, ES10, ES11, ES12, ES13, ES16, ES17) Whatman QMA filter, 720 m3/day (ES09, ES14) / Whatman QMA filter 1632 m3/day (ES01, ES07, ES08) | Daily | Ion chromatography |
| Nitrate PM10 | All | Whatman GF/A filter, 720 m3/day (ES05,ES06, ES10, ES11, ES12, ES13, ES16, ES17) Whatman QMA filter, 720 m3/day (ES09, ES14) / Whatman QMA filter 1632 m3/day (ES01, ES07, ES08) | Daily | Ion chromatography |
| Sum of nitric acid and nitrate | All | NaOH impregnated Whatman 40 filter, 24 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | All | Oxalic acid impregnated Whatman 40 filter, 24 m3/day | Daily | Ion chromatography |
| Ammonium PM10 | ES01, ES07, ES08, ES09, ES14 | High volume sampler | 24 hour, once a week | Ion chromatography |
| Sodium PM10 | ES01, ES07, ES08, ES09, ES14 | High volume sampler | Daily | ICP-AES |
| Calcium PM10 | ES01, ES07, ES08, ES09, ES14 | High volume sampler | Daily | ICP-AES |
| Magnesium PM10 | ES01, ES07, ES08, ES09, ES14 | High volume sampler | Daily | ICP-AES |
| Potassium PM10 | ES01, ES07, ES08, ES09, ES14 | High volume sampler | Daily | ICP-AES |
| Chloride PM10 | ES01, ES07, ES08, ES09, ES14 | High volume sampler | 24 hour, once a week | Ion chromatography |
| Sulphate PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | Ion chromatography |
| Nitrate PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | Ion chromatography |
| Sodium PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | ICP-AES |
| Calcium PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | ICP-AES |
| Magnesium PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | ICP-AES |
| Potassium PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | ICP-AES |
| Ammonium PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | Ion chromatography |
| Chloride PM2.5 | ES01, ES07, ES08, ES12, ES14 | High volume sampler | 24 hour, once a week | Ion chromatography |
| EC/OC PM2.5 | ES01, ES07, ES08, ES12, ES14 | PM2.5 low volume sampler (55 m3/day) | 24 hour, once every 6 days (60 samples per year) | Thermal optical |

| Country: Spain – IDAEA-CSIC (ES1778) |  |  | Main components - EMEP |  | Year: 2023 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Station | Sampling |  | Sampling frequency |  | Analysis method |
| Precipitation |  |  |  |  |  |  |
| Precipitation amount |  |  |  |  |  |  |
| Precipitation amount, official gauge |  |  |  |  |  |  |
| Sulphate |  |  |  |  |  |  |
| Nitrate |  |  |  |  |  |  |
| Ammonium |  |  |  |  |  |  |
| Magnesium |  |  |  |  |  |  |
| Sodium |  |  |  |  |  |  |
| Chloride |  |  |  |  |  |  |
| Calcium |  |  |  |  |  |  |
| Potassium |  |  |  |  |  |  |
| Conductivity |  |  |  |  |  |  |
| pH |  |  |  |  |  |  |
| Air |  |  |  |  |  |  |
| Sulphur dioxide |  |  |  |  |  |  |
| Nitrogen dioxide |  |  |  |  |  |  |
| Nitric acid |  |  |  |  |  |  |
| Ammonia |  |  |  |  |  |  |
| Sulphate | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | ICP-AES |
| Nitrate | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | Ion chromatography |
| Ammonium | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | Ion chromatography |
| Sodium | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | ICP-AES |
| Calcium | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | ICP-AES |
| Magnesium | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | ICP-AES |
| Potassium | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | ICP-AES |
| Chloride | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | Ion chromatography |
| PM10 |  |  |  |  |  |  |
| PM2.5 |  |  |  |  |  |  |
| Sum of nitric acid and nitrate |  |  |  |  |  |  |
| Sum of ammonia and ammonium |  |  |  |  |  |  |
| OC/EC (PM10, PM2.5, PM1) | ES1778R | High vol samplers, DIGITEL PM10/PM2.5, MCV PM1 |  | Four-daily |  | Thermal optical, EUSAAR 2 |

| Country: Sweden |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Nitrate | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Ammonium | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Spectrophotometric, Flow injection analysis |
| Magnesium | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Sodium | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Chloride | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Calcium | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Potassium | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Ion chromatography |
| Conductivity | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | Conductivity meter |
| pH | SE05, SE11, SE12, SE14 | Wet-only | Daily: SE05, SE14; monthly: SE11, SE12 | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | SE05, SE11, SE12, SE14 | KOH-impregnated Whatman 40 filter, 20 m3/day | Daily | Ion chromatography |
| Nitrogen dioxide | SE05, SE11, SE12, SE14 | NaI-impregnated glass sinters, ~0.7 m3/day | Daily | Spectrophotometric, Flow Injection Analysis |
| Nitric acid |  |  |  |  |
| Ammonia |  |  |  |  |
| Sulphate | SE05, SE11, SE12, SE14 | Teflon filter, Mitex membrane, 20 m3/day | Daily | Ion chromatography |
| Nitrate |  |  |  |  |
| Ammonium |  |  |  |  |
| Sodium | SE05, SE11, SE12, SE14 | Teflon filter, Mitex membrane, 20 m3/day | Daily | Ion chromatography |
| Calcium | SE05, SE11, SE12, SE14 | Teflon filter, Mitex membrane, 20 m3/day | Daily | Ion chromatography |
| Magnesium | SE05, SE11, SE12, SE14 | Teflon filter, Mitex membrane, 20 m3/day | Daily | Ion chromatography |
| Potassium | SE05, SE11, SE12, SE14 | Teflon filter, Mitex membrane, 20 m3/day | Daily | Ion chromatography |
| Chloride | SE05, SE11, SE12, SE14 | Teflon filter, Mitex membrane, 20 m3/day | Daily | Ion chromatography |
| PM10 | SE11, SE12 | TEOM (Tapered Element Oscillating Microbalance | Hourly | TEOM |
| PM2.5 | SE11, SE12 | TEOM (Tapered Element Oscillating Microbalance | Hourly | TEOM |
| PM10 | SE05, SE14 | IVL Sampler PModel S10 | Daily | Gravimetric |
| PM2.5 | SE05, SE14 | IVL Sampler PModel S10 | Daily | Gravimetric |
| Sum of nitric acid and nitrate | SE05, SE11, SE12, SE14 | Aerosol filter as for sulphate + KOH-impregnated Whatman 40 filter, 20 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | SE05, SE11, SE12, SE14 | Aerosol filter as for sulphate + Oxalic acid impregnated Whatman 40 filter, 20 m3/day | Daily | Spectrophotometric, Flow injection analysis |

| Country: Switzerland |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) |  |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Nitrate | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Ammonium | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Magnesium | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Sodium | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Chloride | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Calcium | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Potassium | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Ion chromatography |
| Conductivity | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | Conductivity meter |
| pH | CH02, CH05 | Wet-only | Daily (CH02), weekly (CH05) | pH meter |
| Air |  |  |  |  |
| Carbon monoxide | CH01 | Instrumental: CRDS-analyzer | Hourly | Cavity Ring-Down Spectroscopy |
| Sulphur dioxide | CH01, CH02, CH05 | Instrumental: UV-fluorescence | Hourly | UV-fluorescence |
| Nitrogen dioxide | CH01 | Instrumental: Laser Absorption Spectrometer | Hourly | Direct laser absorption |
| Nitrogen dioxide | CH03, CH04, CH53 | Instrumental: Chemiluminescence-monitor | Hourly | Chemiluminescence (molybdenum) |
| Nitrogen dioxide | CH02, CH05 | Instrumental: CAPS-analyzer | Hourly | CAPS |
| Nitrogen monoxide | CH01 | Instrumental: Chemiluminescence-monitor | Hourly | Chemiluminescence (photolytic) |
| Nitrogen monoxide | CH02, CH05, CH53 | Instrumental: Chemiluminescence-monitor | Hourly | Chemiluminescence (molybdenum) |
| Nitric acid | CH02, CH05, CH53 | KOH impregnated Mini-Denuder / modified CEH DELTA-System, 0.5 m3/day | Biweekly | Ion chromatography |
| Ammonia | CH02, CH05, CH53 | Citric acid impregnated Mini-Denuder / modified CEH DELTA-System, 0.5 m3/day | Biweekly | Ion chromatography |
| Sulphate | CH02, CH05 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 720 m3/day | Daily | Ion chromatography |
| Sulphate | CH01 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 1075 m3/day | Monthly | Ion chromatography |
| Nitrate | CH02, CH05, CH53 | KOH impregnated Whatman 1 filter, Delrin filterholder / modified CEH DELTA-System, 0.5 m3/day | Biweekly | Ion chromatography |
| Ammonium | CH02, CH05, CH53 | Citric acid impregnated Whatman 1 filter, Delrin filterholder / modified CEH DELTA-System, 0.5 m3/day | Biweekly | Ion chromatography |
| Sodium | CH02, CH05 | Citric acid impregnated Whatman 40 filter / Schleicher&Schüll filterholder, 18 m3/day | Daily | Ion chromatography |
| Calcium | CH02, CH05 | Citric acid impregnated Whatman 40 filter / Schleicher&Schüll filterholder, 18 m3/day | Daily | Ion chromatography |
| Magnesium | CH02, CH05 | Citric acid impregnated Whatman 40 filter / Schleicher&Schüll filterholder, 18 m3/day | Daily | Ion chromatography |
| Potassium | CH02, CH05 | Citric acid impregnated Whatman 40 filter / Schleicher&Schüll filterholder, 18 m3/day | Daily | Ion chromatography |
| PM10 | CH01 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 1075 m3/day | Daily | Gravimetry |
| PM10 | CH02, CH05 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 720 m3/day | Daily | Gravimetry |
| PM10 | CH03, CH04, CH53 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 720 m3/day | Every 2nd day | Gravimetry |
| PM2.5 | CH02, CH05 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 720 m3/day | Every 4th day | Gravimetry |
| Sum of nitric acid and nitrate | CH02, CH05 | NaOH impregnated Whatman 40 filter / Schleicher&Schüll filterholder, 18 m3/day | Daily | Ion chromatography |
| Sum of ammonia and ammonium | CH02, CH05 | Citric acid impregnated Whatman 40 filter / Schleicher&Schüll filterholder, 18 m3/day | Daily | Ion chromatography |
| OC, EC, TC in PM2.5 | CH02, CH05 | High Volume Samplers, Pallflex XP56 Tissuequartz 2500 QAT-UP, 720 m3/day | Every 12th day | Thermal optical, EUSAAR 2 |

| Country: United Kingdom |  | Main components - EMEP | Year: 2023 |  |
| --- | --- | --- | --- | --- |
|  | Station | Sampling | Sampling frequency | Analysis method |
| Precipitation |  |  |  |  |
| Precipitation amount | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Mass of water collected |
| Precipitation amount, official gauge |  |  |  |  |
| Sulphate | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Nitrate | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Ammonium | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Magnesium | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Sodium | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Chloride | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Calcium | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Potassium | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Ion chromatography |
| Conductivity | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | Conductivity meter |
| pH | GB02, GB06, GB13, GB14, GB15 | Bulk collector | Fortnightly | pH meter |
| Precipitation amount | GB48, GB1055 | Wet only collector | Daily | Mass of water collected |
| Sulphate | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Nitrate | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Ammonium | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Magnesium | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Sodium | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Chloride | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Calcium | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Potassium | GB48, GB1055 | Wet only collector | Daily | Ion chromatography |
| Conductivity | GB48, GB1055 | Wet only collector | Daily | Conductivity meter |
| pH | GB48, GB1055 | Wet only collector | Daily | pH meter |
| Air |  |  |  |  |
| Sulphur dioxide | GB37, GB38, GB43, GB45 | Instrumental | Hourly, 15 minute | UV fluorescence |
| Sulphur dioxide | GB48. GB1055 | Instrumental | Hourly | Online IC |
| Nitrogen dioxide | 12 sites | Instrumental | Hourly | Chemiluminescence |
| Nitrogen monoxide | 12 sites | Instrumental | Hourly | Chemiluminescence |
| Nitric Acid | GB48 | Instrumental | Hourly | Online IC |
| Ammonia | GB48 | Instrumental | Hourly | Online IC |
| PM10 | GB06, GB43, GB48, GB1055 | FDMS | Hourly |  |
| PM2.5 | GB48, GB1055 | FDMS | Hourly |  |
| Ammonium PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Calcium PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Chloride PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Magnesium PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Nitratae PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Potassium PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Sodium PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |
| Sulphate PM10, PM2.5 | GB48, GB1055 | Instrumental | Hourly | Online IC |

————————————————————————————————————————————————