EMEP/CCC-Report 4/2025

# VOC measurements 2023

Authors: Sverre Solberg (NILU), Anja Claude (Deutscher Wetterdienst) and Stefan Reimann (EMPA)

EMEP/CCC-Report 4/2025

EMEP Co-operative programme for monitoring and evaluation of the long-range transmission of air pollutants in Europe

VOC measurements 2023

Sverre Solberg1, Anja Claude2 and Stefan Reimann3

Affiliation:

- Stiftelsen NILU, Kjeller, Norway

- Deutscher Wetterdienst, Hohenpeissenberg, Germany

- Swiss Federal Laboratories for Materials Science and Technology (Empa), Dübendorf, Switzerland

|  | NILU, P.O. Box 100, NO-2027 Kjeller, Norway Chemical Co-ordinator Centre of EMEP (CCC) |
| --- | --- |

EMEP/CCC-Report 4/2025

NILU
P.O. Box 100
NO-2027 Kjeller,
Norway

Chemical Co-ordinator Centre of EMEP (CCC)

Phone +47 63 89 80 00
nilu@nilu.no
www.nilu.no

Stiftelsen NILU

Org.no. 941 705 561

Certifications

NS-EN ISO 9001
NS-EN ISO 14001

Accreditation

NS-EN ISO/IEC 17025

ISBN 978-82-425-3174-2

ISSN 2464-3920

Cover picture:

EMEP

Title

VOC measurements 2023

Norwegian title

VOC-målinger 2023

Author(s)

Sverre Solberg (NILU), Anja Claude (Deutscher Wetterdienst) and Stefan Reimann (EMPA)

Short summary (English)

This report documents the EMEP VOC monitoring carried out in 2023. The levels of the measured species in 2023 are presented as well as the 2023-status and history of the VOC programme. The geographical pattern of the species in Europe is discussed as well as the long-term trend during the last 20 years.

Short summary (Norwegian)

Denne rapporten dokumenterer EMEP VOC-overvåkingen utført i 2023. Nivåene av de målte komponentene i 2023 presenteres sammen med status i 2023 og historikken for VOC-programmet. Den geografiske fordelingen av komponentene i Europa diskuteres samt den langsiktige trenden de siste 20 årene.

| Keywords |  |
| --- | --- |
| EMEP, VOC, NMHC, monitoring |  |
| Number of pages | Classification |
| 65 | A-Open |
| Project leader | NILU project number |
| Wenche Aas | O-7726 |
| Client/Report prepared for | Contact reference |
| United Nations Economic Commission for Europe (UNECE); https://projects.nilu.no/ccc/reports.html | MoU ECE/ENV/LRA/2011/001 |
| Signature of the responsible person | Date |
| Name, title | …………………. |

© Stiftelsen NILU

Citation

Solberg, S., Claude, A. and Reimann, S. (2025). VOC measurements 2023 (EMEP/CCC-report 4/2025). Kjeller: NILU.

Contents

Summary	5

1	Introduction	6

1.1	Historical background	6

2	Status of the measurement programme	7

2.1	The station network	7

3	VOC concentrations in 2023	13

3.1	General levels	13

3.2	Regional distribution of VOC	21

4	Long-term trends in VOC	25

5	Acknowledgement	32

6	References	32

Appendix 1 Time series of daily means of VOCs measured in 2023	35

# Summary

This report presents VOC (volatile organic compound) measurements carried out during 2022 at EMEP monitoring sites. In total, 19 sites reported VOC-data from EMEP VOC sites this year.

The monitoring of VOC has become more diverse with time in terms of methods and instrumentation. Starting in the early 1990s with standardized methods based on manual sampling in steel canisters and adsorption tubes with subsequent analyses at the lab, the methods now consist of a variety of instruments and measurement principles, including automated continuous monitors and manual flask samples.

Within the EU infrastructure project ACTRIS, data quality issues related to measurements of VOC are important topics. Most of the institutions providing VOC-data to EMEP are participating in the ACTRIS infrastructure, either as formal partners or on a voluntary basis. Participation in ACTRIS means an extensive effort with data-checking including detailed discussions between the ACTRIS community and individual participants. There is no doubt that this extensive effort has benefited the EMEP-program substantially and has led to improved data quality in general.

In general, the levels in 2022 agree well with the previous 5-years period, but the lack of sites with long and consistent time series poses a substantial problem for making reliable long-term trend assessments of the VOC. Breaks in the time series are caused by shifts in instrumentation/method or stations being closed or moved etc. In this report, we have estimated the trends in individual NMHCs over the 2003-2022 period for one single station (Hohenpeißenberg), using a so-called GAM (generalized additive model), that removes the effect of anomalies in the meteorology.

The results indicate substantial reductions in the observed levels of the NMHCs over the 2003-2022 period at this site. Furthermore, significant differences in the trends for the various species are found. Strongest declines are found for benzene, toluene, ethene, and m-p-xylene which according to the AirGAM model dropped 55-65 % during this 20 year’s period. Next to this group, ethyne and i-pentane dropped around 45 % whereas for the other NMHCs a drop of the order of 10-30 % is seen. These trend numbers refer to the trends adjusted for variations in meteorology.

These reductions are in line with the emission data from the so-called EMEP-West region although a quantitative comparison species by species could not be done since the EMEP emission data are given for the sum of NMHC and not for individual substances.

In July 2022 a one-week intensive measurement campaign for VOC was carried out linked to a European heat wave. A large number of European institutions and stations were involved and contributed with an extensive suite of measurements from passive samplers and online monitors of various kinds.

VOC measurements 2023

## Introduction

### Historical background

The EMEP VOC monitoring programme was initiated at the EMEP Workshop on Measurements of Hydrocarbons/VOC in Lindau, 1989 (EMEP/CCC, 1990). A three-fold objective of the measurement programme was defined at the workshop:

- Establishing the current ambient concentrations

- Compliance monitoring (“Do the emission control programme lead to a reduction of atmospheric concentrations?”)

- Support to the transboundary oxidant modelling (prognostic and diagnostic)

The Workshop recommended that as a first step it would be sufficient with VOC monitoring at 10-15 rural sampling sites and taking two samples per week centred at noon GMT at each station. Collection in stainless steel canisters and analyses by high resolution gas chromatography was recommended for the detection of light hydrocarbons, whereas impregnated adsorbent tubes sampling combined with high performance liquid chromatography (HPLC) was recommended for the detection of carbonyls.

VOC measurements within EMEP started with the collection of grab samples of light hydrocarbons in mid-1992 and measurements of carbonyls in 1993. Initially, five stations were included in the monitoring programme: Rucava (LV0010), Košetice (CZ0003), Waldhof (DE0002), Tänikon (CH0032) and Donon (FR0008). Since then, the number and selection of VOC measurement sites have changed several times.

EMEP VOC measurements are reported annually and presented in reports for consideration by EMEP-TFMM and the EMEP Steering Body. Previous results from the EMEP VOC programme have been presented in annual reports (e.g. Solberg, 2023 and references therein). An EMEP expert meeting on VOC measurements was organised in Berlin, 1994 (EMEP/CCC, 1995), and an evaluation of the measurement programme was made in 1995 (Solberg et al., 1995).

VOC-data from the EMEP-network have been published and documented in numerous publications, e.g. Waked et al. (2016), Hellen et al. (2015), Hoerger et al. (2015), Malley et al. (2015), Tørseth et al. (2012), Worton et al. (2012), Sauvage et al. (2009), Plass-Dülmer et al. (2009), Plass-Dülmer et al. (2002), Hakola et al. (2006), Borbon et al. (2004), Solberg et al. (2001), and Solberg et al. (1996). Details about the VOC monitoring program are given in the EMEP annual reports such as Solberg (2024) and references therein.

## Status of the measurement programme

### The station network

The locations of the EMEP monitoring sites for VOC in 2023 are shown in Figure 1 and an overview of the measurement programme and the responsible laboratories is given in Table 1. In total, 18 measurement sites and 11 laboratories were included in the EMEP VOC program in 2023.

The measured VOCs consist of various groups of species which could be split into non-methane hydrocarbons hereafter named NMHC and oxygenated species hereafter named OVOC. Monitoring of NMHC is carried out at all sites except ES0001, whereas OVOC are measured at fewer sites.

Figure 1:	Monitoring sites for VOC in 2023.

The VOC monitoring at EMEP sites has become more diverse with time in terms of methods and instrumentation. This is illustrated in Figure 2. The EMEP VOC monitoring programme started in 1992 with one standardized method for NMHC based on manual sampling in steel canisters and subsequent lab analyses and another standardized method for sampling of OVOCs by DNPH tubes followed by analyses with HPLC in the lab. During the years is a variety of instruments and measurement principles have been applied, including PTR-MS, Medusa monitors, specialized online GC monitors and adsorption tubes for aromatic species.

Figure 2:	History of the VOC monitoring as a function of chemical group (left) and instrument (right).

The types of VOC species have also expanded from C2-C6 NMHC and a few light OVOCs in the start of the period to a very wide spectrum of species including terpenes, alcohols, C7-C9 NMHC, acetonitrile and numerous branched saturated and unsaturated NMHC as given in the Appendix.

For OVOC the original EMEP method based on sampling in DNPH adsorption tubes with subsequent lab analyses is still the method used at one site in Spain (ES0001). In addition, OVOC are also measured by PTR-MS and by the GC-GC FID/FID system at Beromünster in Switzerland.

Although many sites have contributed to the EMEP VOC programme since the early 1990s, very few sites have long and continuous time series. Additionally, shifts in instrumentation imply breaks in the time series. At some sites these shifts are a matter of upgrading the GC monitor, with minor effects on the measured values, while at other sites they represent significant breaks in the data time series. The VOC network has also changed due to stations being closed or moved (like e.g. CH0005 that was replaced with CH0053 in 2018).

The history of the VOC station network is illustrated in Figure 3 showing the monitoring at the various sites as a function of time since the onset in 1992. The changing monitoring network with few sites having a long-term continuous time series is clearly seen from this figure and poses a major obstacle for doing long-term trend studies.

Figure 3:	Monitoring history as a function of site 1992-2023.

All VOC data are reported annually to EBAS at NILU/EMEP-CCC through the EBAS submission tool (https://ebas-submit-tool.nilu.no/). For sites taking part in ACTRIS (see Table 1), the initial data submission is followed by a rigorous data quality checking procedure through an issue tracker (https://mantis.nilu.no/), involving a dedicated QA tool that is based on ratios of species (e.g. ethane:propane) and comparisons of the levels at different sites. Based on the results from this tool, the ACTRIS topic centre adds comments and questions to the data series followed by a discussion with the data provider and NILU/EMEP-CCC regarding outliers, unexpected shifts in the data etc and then, finally, which data to invalidate or flag. Since this discussion is done in an issue tracker, the history of bugs, outliers and breaks in the time series is easily available at a later stage.

This QA system has been in operation for some years and have clearly led to an improvement in the overall data quality of the part of the monitoring program included in ACTRIS.  The ACTRIS community is open to include non-ACTRIS sites in this exercise which has been the case for e.g. the sites run by UBA in Germany.

An example of the QA tool for data from DE0054 (Zugspitze) and DE0043 (Hohenpeissenberg) is given in Figure 4, showing submitted data for ethene and ethyne for 2023 together with previous years of data. The marked outliers in the plot were inspected by the data providers.

Figure 4:	Example of the  so-called  «@VOC@ tool» used for quality checking of the submitted  NMHC data  within ACTRIS. This example shows data from  DE0054 (Zugspitze) on top and DE0043 (Hohenpeissenberg) on the bottom as copied from the issue tracker

The EMEP VOC monitoring sites are by definition located in rural background areas but in practice the sites are exposed to more nearby emissions at a varying extent. The data from Chilbolton observatory, located in southern England, reflect the influence of populated areas in the whole of southern England, including road traffic emissions, and show a number of short-term (hourly) spikes in the data. Data from Auchencorth Moss in Scotland, a rural location around 20 km south of Edinburgh also show very spiky hourly time series with peak levels of propane, n-butane and other species. The high concentrations at that site are mainly seen during periods with winds from the northern sector, pointing to fresh anthropogenic emissions from the urban area around Edinburgh as well as from petroleum refineries in the same direction. It turns out that for both these sites, most of the spikes are of short duration while daily average levels seem fine.

NMHC data for the sites from UBA in Germany, i.e. Waldhof, Schauinsland, Neuglobsow, Schmücke, Zingst, and Zugspitze have been subject to problems in 2023 as in previous years. This concerns alkenes with three or more C-atoms (propene and higher) but also aromatics (o-xylene) and some higher alkanes. Dubious data have been invalidated by NILU in 2023 as in preceding years. Due to late data delivery, data from the UBA sites for 2023 could not be included in the ACTRIS QA procedure.

As in previous years, NMHC data from San Pablo (ES0001) for 2023 have been submitted to NILU but are not included in this report. The reason for this is, as discussed in previous annual reports, that the data show levels that differ substantially from expected values indicating major problems with the sampling or chemical analyses.  There is an ongoing dialogue between NILU and MMA about this issue, and a shift in technical equipment (pump etc) and in localisation of the sampling are promising changes carried out at the site.

Table 1:	VOC monitoring at EMEP sites in 2023. The columns give the station names, site code, and the sampling frequencies for hydrocarbons (HC) and carbonyl compounds (Carb). The institute responsible for the chemical analyses is also given. Whether the station is part of the ACTRIS-2 project is also indicated.

| Station | Code | HC1) | Institute2) | Carb1) | Method | ACTRIS | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Zeppelin Mtn. | NO0042 | cont | NILU | - | MEDUSA | y |  |
| Pallas | FI0096 | cont | FMI | - | GC/MS | y |  |
| Hyytiälä | FI0050 | cont | UHel | - | PTR-MS | y |  |
| Auchencorth Moss | GB0048 | cont | Ricardo | - | GC/MS | y |  |
| Chilbolton Obs. | GB1055 | cont | Ricardo | - | GC/MS | y |  |
| Waldhof | DE0002 | reg | UBA | - | canisters | n | twice/week |
| Schauinsland | DE0003 | reg | UBA | - | “ | n | “ |
| Neuglobsow | DE0007 | reg | UBA | - | “ | n | “ |
| Schmücke | DE0008 | reg | UBA | - | “ | n | “ |
| Zingst | DE0009 | reg | UBA | - | “ | n | “ |
| Zugspitze | DE0054 | reg | UBA | - | “ | n | “ |
| Hohenpeissenberg | DE0043 | daily | DWD | - | GC/FID | y | twice/day |
| Košetice | CZ0003 | reg | CHMI | - | canisters | y | twice/week |
| Jungfraujoch | CH0001 | cont | EMPA | - | MEDUSA | y |  |
| Beromünster | CH0053 | cont | EMPA | - | GC/FID | y |  |
| SIRTA | FR0020 | cont | LSCE-CNRS-CEA |  | PTR-MS | y |  |
| Mt. Cimone | IT0009 | cont | UU | - | GC/MS | y |  |
| San Pablo | ES0001 | - | MMA | reg | DNPH | n | twice/week |

1)	reg			= regularly (2-3 samples per week), cont = continuously

2)	CHMI			= Czech Hydrometeorological Institute

DWD			= Deutscher Wetterdienst

IMT LD			= Institut Mines Telecom Lille Douai

EMPA			= Swiss Federal Lab. for Materials  Testing and Research

FMI			= Finnish Meteorological Institute

UHel			= Univ. Helsinki

UBA			= Umweltbundesamt (Germany)

UU			= University of Urbino

MMA			= Minestrio de Medio Ambiente

LSCE-CNRS-CEA 	= Laboratoire des Sciences du Climat et de l'Environnement

CNRS-LaMP 		= Laboratoire de Meteorologie Physique

## VOC concentrations in 2023

### General levels

Time series plots of the diurnal medians of all compounds at all stations during 2023 are given in the Appendix. Figure 5 shows the spread of data values for all stations and species in 2023 in box and whisker plots together with the corresponding data during the preceding 5-years period (2018-2022). The sites are arranged from north to south going from left to right in the panels. Thus, the panels in Figure 2 indicate both the north-to-south differences, the deviation of the 2023 concentration levels relative to the previous five years’ climatology as well as the spread in 2023 data at each site separately.

In general, the 2023 data show similar mean levels and distribution compared to previous years with some exceptions. Data from CZ0003 (Kosetice) are examples of such exceptions, where the levels of some aromatics (ethylbenzene, m+p-xylene, o-xylene, toluene) and n-heptane show higher levels than seen in the previous 5 years’ period.

For FR0020 (SIRTA) the amount of EBAS data in the years before 2023 are scattered, so the discrepancies seen when comparing 2023 to the historical data could be hampered by poor data capture. Apart from that, the levels in 2023 agree very well with the levels in the previous 5-years period in general although with a few exceptions.

Figure 5:	Box- and whisker-diagrams for VOCs based on daily median levels in 2023 (blue) compared to the levels during 2018-2022. The boxes enclose the 25- and 75-percentile with the median marked inside.

Figure 5 (cont.).

Figure 5 (cont.).

Figure 5 (cont.).

Figure 5 (cont.).

Figure 5 (cont.).

Figure 5 (cont.).

### Regional distribution of VOC

Figure 6 shows maps with the stations’ annual median concentrations of VOCs in 2023. Note that since the steel canisters are all sampled at daytime (normally at noon), a bias could be inherent in these plots when compared with the medians of the 24 h based data from online GCs. A bias for other species is also likely to be seen at a varying extent. Some of the mountain stations (Zugspitze, Hohenpeissenberg and Mt Cimone) are influenced by diurnal venting of the planetary boundary layer and will receive upslope polluted air masses at daytime when the vertical mixing is sufficiently strong and cleaner free tropospheric air masses at night. The station at Jungfraujoch (3578 m asl) will on the other hand most of the time be located in the free troposphere, above the planetary boundary layer.

Figure 6:	Annual median concentration of VOCs in 2023.

Figure 6 (cont.).

Figure 6 (cont.).

## Long-term trends in VOC

According to the official emission data, there have been marked reductions in anthropogenic emissions of VOCs during the last decades in Europe. Overview tables with reported emission trends for individual countries have been published on the CEIP website at https://www.ceip.at/. Detailed information on the sectoral level can also be accessed in WebDab.

There are substantial differences in the emission trends between countries and regions. For the area defined as “EMEP-West”, there has been an overall reduction in VOC emissions of more than 40 % for the period 2000-2020 (Fagerli et al., 2022) and for individual countries such as Germany, France, UK, Italy, Spain, and Poland, the reductions have been 43%, 54%, 56%, 46%, 38% and 17%, respectively, as given in the same report.

For the area defined as “EMEP-East”, however, the emission data including so-called gap-filling indicate a nearly flat development from 2000 to 2020 for NMVOC. As stated in last year’s EMEP Status report (Fagerli et al., 2022), the emission estimates for EMEP-East are, however, much more uncertain than the data for EMEP-West.

Declines in the measured concentrations of hydrocarbons have been reported from suburban/urban sites at several locations. Based on a network of high-frequency continuous monitoring of C2-C8 hydrocarbons in the UK, mostly at urban/suburban locations, Derwent et al. (2014) found substantial declines in concentrations with recent levels close to an order of magnitude below the levels in the early 1990s. They estimated exponential declines in concentrations of the order of -11% y-1 to -22% y-1 for the period 1994-2012. They also found a marked difference between ethane and propane on one side which showed relatively stable levels, while other alkanes showed pronounced declines.

Long-term monitoring data from an urban network in Switzerland (Hüglin, pers. comm.) also show strong declines in the concentrations of NMHC and OVOC from the start of the 1990s to the present.

Various trend studies have been carried out for VOC-data from EMEP rural sites as well. Sauvage et al. (2009) and Waked et al. (2016) found clear decreases at the French EMEP sites of most NMHCs. Ethane was an exception to this and showed more stable levels.

Analyses of the twenty years NMHC monitoring at the EMEP/GAW site Pallas in Northern Finland revealed a significant downward trend only for ethyne (Hellen et al., 2015). They concluded that other source regions than the EU were dominating the NMHC levels at the site. Based on source area estimates, they found that the Eastern parts of the continent were the main source regions for high concentrations at Pallas.

A simple 1:1 relationship between observed VOC concentrations at rural background sites and the overall European emission numbers is not to be expected. Interannual variations in atmospheric transport patterns, vertical mixing, photochemical oxidation as well as spatial differences in emission reductions complicate the analyses.

Furthermore, various procedures for trend analyses in terms of mathematical methods, selection of time periods and stations etc. could give different results. In previous VOC annual reports (Solberg et al., 2022) daily measurements of selected VOCs at five EMEP sites were analyzed with two separate statistical methods: The method used for trace gases in the AGAGE project (Simmonds et al., 2006) as well as best-fit seasonal trend curves calculated by non-linear least squares fit using a standard statistical package (Markwardt, 2009). In addition to these two methods the Mann-Kendall/Theil-Sen’s slope methodology (MK) was applied to the annual median concentrations (Bronaugh and Werner, 2019; Sen, 1968).

In the present report a GAM (generalized additive model) has been used based on the AirGAM model (Walker et al., 2023). This is an air quality trend and prediction model developed at NILU in cooperation with the European Environment Agency (EEA). AirGAM is based on nonlinear regression and is capable of estimating trends in daily measured pollutant concentrations at air quality monitoring stations, discounting for the effects of trends and time variations in corresponding meteorological data.

AirGAM has been applied to daily levels of hydrocarbons measured at Hohenpeissenberg (DE0043) during the period 2001-2022 together with daily values of temperature, humidity, wind speed, wind direction, and mixing height as compiled from ECMWF met data in the same way as described in Walker et al. (2023). The same model set-up as used for NO2 (Solberg et al., 2021) was used for these hydrocarbons, implying that daily mean values of all the meteorological covariates were used in the regression and furthermore, that a log-based approach was assumed, i.e. the logarithm of the daily hydrocarbon levels was used as the dependent variable.

The trend during 2001-2023 in the relative concentration levels of the NMHCs with 2001 being the reference year is shown in Figure 7 while Figure 8 shows the daily mean levels in 2023 as observed and predicted with the AirGAM model. Please note that whereas the measurements are done twice a day (noon and midnight) the meteorological data input to AirGAM has been based on the daily averages of the values at 00, 06, 12 and 18.

Figure 7:	Meteorological adjusted trend in relative concentrations of NMHCs at Hohenpeissenberg 2003-2023 as calculated by the AirGAM model (Walker et al., 2023).

The results shown in Figure 7 indicate a substantial reduction in the NMHCs measured at DE0043 and, furthermore, significant differences in the trends for the various species. Strongest declines are found for toluene, ethene, m-p-xylene and i-pentane which in 2023 are seen at levels only 35-45% of that in 2001 meaning a drop of the order of 55-65 % since 2001. The other NMHC have dropped to around 70-90% of the 2001 levels meaning a reduction of 10-30% during this 22-years period.

These marked reductions in observed NMHC levels in Europe since 2001 are in line with the emission data from the EMEP-West region as mentioned above. A quantitative comparison species by species could not be done since the EMEP emission data are given for NMHC as a whole and not for individual substances.

The reliability of the AirGAM model could be judged from Figure 8 showing the agreement between the observed and modelled daily mean values during 2023 for 9 anthropogenic NMHCs from ethane to toluene plus isoprene including some statistical metrics for the data. Unfortunately, there was a long period of missing observations in the summer season (June-September) at Hohenpeissenberg in 2023.

It is important to keep in mind that AirGAM is a statistical model (or tool) that is based purely on observed relationships between air quality data and meteorological parameters without any parameterization of physical or chemical processes. This makes it well suited for analysing historical data while the model could of course not be used for predicting the effects of emission changes. The results in Figure 8 show good agreements between observed and predicted daily values in 2023 although some spikes in the observed data are not reproduced by AirGAM.

Figure 8:	Daily mean concentrations of selected NMHCs as observed (red) and modelled by AirGAM (blue) at Hohenpeissenberg in 2023. Statistical metrics as the correlation coefficient (r), normalized mean bias (NMB), normalized mean gross error (NMGE), mean observed (mOBS) and mean modelled (mMOD) level are given in the panels.

Figure 12 (cont.).

Figure 12 (cont.).

Figure 12 (cont.).

## Acknowledgement

Data originators for individual datasets can be found as part of the metadata by visiting http://ebas.nilu.no. A special thanks to the providers of data for 2023: Heidi Hellen (FMI), Ilona Ylivinkka (UHEL), Lucie Školoudova (CHMI), James Dernier (Ricardo), Bryan Hellack (UBA), Therese Salameh (IMT), Norbert Schmidbauer (NILU), Arduini Jgor (UU), and Fernandez Monistrol Jose Antonio (MMA).

## References

Bronaugh, D. and Werner, A. (2019) zyp: Zhang + Yue-Pilon Trends Package. URL: https://CRAN.R-project.org/package=zyp, r package version 0.10-1.1, 2019.

Derwent, R. G., Dernie, J. I. R., Dollard, G. J., Dumitrean, P., Mitchell, R. F., Murrells, T. P., Telling, S. P., Field, R. A. (2014) Twenty years of continuous high time resolution volatile organic compound monitoring in the United Kingdom from 1993 to 2012. Atmos. Environ., 99, 239-247.

EMEP/CCC (1990) EMEP workshop on measurement of hydrocarbons/VOC. Lindau, Federal Republic of Germany. Lillestrøm, NILU (EMEP/CCC Report 3/90).

EMEP/CCC (1994) Expert meeting on EMEP VOC measurements Berlin, Germany, 30 November - 2 December 1994. Kjeller, NILU (EMEP/CCC-Report 6/95). URL: https://projects.nilu.no/ccc/reports/cccr6-95.pdf (accessed 16.10.2024).

EMEP/CCC (2014) Manual for sampling and chemical analysis. Kjeller, NILU (EMEP/CCC-Report 1/2014).

Fagerli, H., Benedictow, A.M.K.,  van Caspel, W., Gauss, M., Ge, Y., Jonson, J. E., Klein, H., Nyiri, A., Simpson, D. Tsyro, S., Valdebenito Bustamante, A.M.,  Wind, P., Aas, W., Hjellbrekke, A.-G., Solberg, S., Tørseth, K., Yttri, K.E., Matthews, B., Schindlbacher, S., Ullrich, B., Wankmüller, R., Klimont, Z., Scheuschner, T., Kuenen, J., Hellen, H., Jaffrezo, J.-L., Tusha, D., Mothes, F., Salameh, T., van Droge, B. L., Wegener, R. (2023) Transboundary particulate matter, photo-oxidants, acidifying and eutrophying components. Status report 2022. Oslo, Norwegian Meteorological Institute (EMEP Report 1/2023).

Hakola, H., Hellén, H., Laurila, T. (2006) Ten years of light hydrocarbons (C2 C6) concentration measurements in background air in Finland. Atmos. Environ., 40, 3621-3630.

Hellén, H., Kouznetsov, R., Anttila, P., Hakol, H. (2015) Increasing influence of easterly air masses on NMHC concentrations at the Pallas-Sodankylä GAW station. Boreal Env. Res., 20, 542-552.

Hoerger, C. C., Claude, A., Plass-Duelmer, C., Reimann, S., Eckart, E., Steinbrecher, R., Aalto, J., Arduini, J., Bonnaire, N., Cape, J. N., Colomb, A., Connolly, R., Diskova, J., Dumitrean, P., Ehlers, C., Gros, V., Hakola, H., Hill, M., Hopkins, J. R., Jaeger, J., Junek, R., Kajos, M. K., Klemp, D., Leuchner, M., Lewis, A. C., Locoge, N., Maione, M., Martin,D., Michl, K., Nemitz, E., O’Doherty, S., Perez Ballesta, P., Ruuskanen, T. M., Sauvage, S., Schmidbauer, N., Spain, T. G., Straube, E., Vana, M., Vollmer, M. K., Wegener, R., Wenger, A. (2015). ACTRIS non-methane hydrocarbon intercomparison experiment in Europe to support WMO GAW and EMEP observation networks. Atmos. Meas. Tech., 8, 2715-2736, doi:10.5194/amt-8-2715-2015.

Malley, C. S. Braban, C. F., Dumitrean, P., Cape, J. N., Heal, M. R. (2015) The impact of speciated VOCs on regional ozone increment derived from measurements at the UK EMEP supersites between 1999 and 2012. Atmos. Chem. Phys., 15, 8361-8380, doi:10.5194/acp-15-8361-2015.

Markwardt, C. B. (2009) Non-linear least squares fitting in IDL with MPFIT. In: Astronomical Data Analysis Software and Systems XVIII. Proceedings of a workshop held at Hotel Loews Le Concorde, Quebec City, QC, Canada, 2-5 November 2008. Ed. by: Bohlender, D., Dowler, P. & Durand, D. San Francisco, Astronomical Society of the Pacific (ASP Conference Series, Vol. 411). pp. 251-254.

Plass-Dülmer, C., Michl, K., Ruf, R., Berresheim, H. (2002) C2-C8 hydrocarbon measurement and quality control procedures at the Global Atmosphere Watch Observatory Hohenpeissenberg. J. Chromatogr., 953, 175-197.

Plass-Dülmer, C., Schmidbauer, N., Slemr, J., Slemr, F., D’Souza, H. (2006) European hydrocarbon intercomparison experiment AMOHA part 4: Canister sampling of ambient air. J. Geophys. Res., 111D, 4306, doi:10.1029/2005jd006351.

Sauvage S., Plaisance, H, Locoge, N., Wroblewski, A., Coddeville, P., Galloo, J. (2009) Long term measurement and source apportionment of non-methane hydrocarbons in three French rural areas. Atmos. Environ., 43, 2430-2441.

Simmonds, P. G., Manning, A. J., Cunnold, D. M., McCulloch, A., O’Doherty, S., Derwent, R. G., Krummel, P. B., Fraser, P. J., Dunse, B., Porter, L. W., Wang, R. H. J., Greally, B. R., Miller, B. R., Salameh, P., Weiss, R. F., and Prinn, R. G. (2006) Global trends, seasonal cycles, and European emissions of dichloromethane, trichloroethene, and tetrachloroethene from the AGAGE observations at Mace Head, Ireland, and Cape Grim, Tasmania, J. Geophys. Res., 111, D18304, doi:10.1029/2006JD007082.

Solberg, S., Dye, C., Schmidbauer, N., Herzog, A., Gehrig, R. (1996) Carbonyls and nonmethane hydrocarbons at rural European sites from the Mediterranean to the Arctic. J. Atmos. Chem., 25, 33-66.

Solberg, S., Dye, C., Walker, S.-E., Simpson, D. (2001) Long-term measurements and model calculations of formaldehyde at rural European monitoring sites. Atmos. Environ., 35, 195-207.

Solberg, S., Walker, S.-E., Schneider, P., and Guerreiro, C. (2021) Quantifying the Impact of the Covid-19 Lockdown Measures on Nitrogen Dioxide Levels throughout Europe. Atmosphere, 12, 131, doi:10.3390/atmos12020131.

Solberg, S., Claude, A., Reiman, S., Sauvage, S., Walker, S.-E. (2022) VOC measurements 2020. Kjeller, NILU (EMEP/CCC-Report 4/2022).

Tørseth, K., Aas, W., Breivik, K., Fjæraa, A.M., Fiebig, M., Hjellbrekke, A. G., Lund Myhre, C., Solberg, S., Yttri, K. E. (2012) Introduction to the European Monitoring and Evaluation Programme (EMEP) and observed atmospheric composition change during 1972–2009. Atmos. Chem. Phys., 12, 5447-5481, doi:10.5194/acp-12-5447-2012.

Walker, S.-E., Solberg, S., Schneider, P., and Guerreiro, C. (2023) The AirGAM 2022r1 air quality trend and prediction model. Geosci. Model Dev., 16, 573–595, doi:d10.5194/gmd-16-573-2023.

Waked, A., Sauvage, S., Borbon, A., Gauduin, J., Pallares, C., Vagnot, M. P., Thierry, L., and Locoge, N. (2016) Multi-year levels and trends of non-methane hydrocarbon concentrations observed in ambient air in France. Atmos. Environ., 141, 263-275, doi:10.1016/j.atmosenv.2016.06.059.

Worton, D. R., Sturges, W. T., Reeves, C. E., Newland, M. J., Penkett, S. A, Atlas, E., Stroud, V., Johnson, K., Schmidbauer, N., Solberg, S. (2012) Evidence from firn air for recent decreases in non-methane hydrocarbons and a 20th century increase in nitrogen oxides in the northern hemisphere. Atmos. Environ., 54, 592-602.

# Appendix 1
Time series of daily medians of VOCs measured in 2023

————————————————————————————————————————————————